The program used for the ADC display and the LCD display are:

#include "msp430f5438.h"

#include <intrinsics.h>

#include "hal_lcd.c"

#include "hal_lcd_fonts.c"

#define   C_name           (0)

#define   C_fill           (1)

#define   C_space          (2)

#define   C_thousands      (3)

#define   C_hundreds       (4)

#define   C_tens           (5)

#define   C_ones           (6)

volatile long temp;

char Data[7];

void t_int ();

void _delay_ms(unsigned int ms);

unsigned char adc_read();

void segment7_putnum(unsigned int Num);

unsigned int adc_out;

unsigned int l_read();

void ldcstatus();

void main(void)

{

WDTCTL = WDTPW + WDTHOLD;

P9OUT |= 0x02;                            // Set P9.1 to output direction

P9SEL |= 0x0F;                            // P9.1,2,3 option select

t_int ();

_delay_ms(5000);

halLcdInit();

halLcdClearScreen();

halLcdPrintLine("VEC_LAB",1,0x04);

_delay_ms(2000);

ldcstatus();

halLcdClearScreen();

halLcdPrintLine("Tank Height",2,0x04);

_delay_ms(4000);

halLcdPrintLineCol("adcvalue",4,8,0x04);

_delay_ms(2000);

//ADC initilization

unsigned int tp1;

P6SEL |= 0x80;

P1OUT |= 0x01;

_delay_ms(1000);

REFCTL0 |= REFMSTR+REFVSEL_2+REFON+REFTCOFF;

ADC12CTL0 = ADC12SHT0_7+ADC12REFON;                                // Set sample time to 192 cycles

ADC12CTL0 = ADC12ON;

ADC12CTL1 = ADC12CONSEQ_0 + ADC12SSEL_1 + ADC12SHS_0; // Enable single channel & sample

ADC12CTL1 = ADC12CSTARTADD_0;                        // timer

ADC12CTL2 = ADC12REFOUT;

ADC12MCTL0 = ADC12SREF_1 + ADC12INCH_7;            // ADC input ch A7 => sensor input

ADC12IE = 0x01;

_delay_ms(75);

ADC12CTL0 |= ADC12ENC;

P1OUT &=~0x01;

P1OUT |= 0x02;

_delay_ms(2500);

do

{

ADC12CTL0|= ADC12SC;

halLcdClearScreen();

halLcdPrintLine("Height in Cms",3,0x04);

_BIS_SR(LPM3_bits+GIE);

__no_operation();

_delay_ms(500);

P1OUT |=0x01;

tp1 = temp;                              // Read the ADC Channel 7 data

ldcstatus();                            //chk LCD Busy

segment7_putnum(tp1);

_delay_ms(2500);                        //Delay

}  while(1);                           // Continously read the ADC Channel 7

}

void t_int ()

{

TB0CCR1 = 1000;

TB0CTL = TASSEL_0 + MC_2 +TBCLR;              // ACLK, continuous mode

TB0CCTL0 = CCIE;                            // CCR0 interrupt enabled

}

void _delay_ms(unsigned int ms)

{

unsigned int i,j;

for ( j = 0; j < ms; j++)

{

for (i = 0; i <= 5; i++);            // 1 millisecond

}

}

void ldcstatus()

{

while (UCB2STAT & UCBUSY);

LCD_CS_RST_OUT |= LCD_CS_PIN;          //CS = 1 --> Stop Transfer

LCD_CS_RST_OUT &= ~LCD_CS_PIN;         //CS = 0 --> Start Transfer

}

void segment7_putnum(unsigned int Num)

{

//unsigned char adc;

unsigned char thousands = '0';

unsigned char hundreds = '0';

unsigned char tens = '0';

unsigned char ones = '0';

unsigned int result;

char Data[8];

result = Num;

while (result >= 1000){                 // Calculate hundreds' place

thousands++;

result -= 1000;

}

if (thousands == '0') thousands = ' ';    //eliminate preceding zero, if any

while (result >= 100){                  // Calculate tens' place

hundreds++;

result -= 100;

}

if (hundreds== '0') hundreds = ' ';    //eliminate preceding zero, if any

while (result >= 10){                  // Calculate tens' place

tens++;

result -= 10;

}

ones += result;                        // Remainder is ones' place

Data[C_name] = 'H';

Data[C_fill] = 'V';

Data[C_space] = '=';

Data[C_thousands] = thousands;

Data[C_hundreds] = hundreds;

Data[C_tens] = tens;

Data[C_ones] = ones;

ldcstatus();

halLcdPrintLine(Data,4,0x04);

}

// ADC interrupt routine

#pragma vector = ADC12_VECTOR

__interrupt void ADC12_ISR(void)

{

switch(__even_in_range(ADC12IV,34))

{

case  0: break;                           // Vector  0:  No interrupt

case  2: break;                           // Vector  2:  ADC overflow

case  4: break;                           // Vector  4:  ADC timing overflow

case  6:                                  // Vector  6:  ADC12IFG0

temp = ADC12MEM0;

__bic_SR_register_on_exit(LPM3_bits);   // Exit active CPU

break;

case  8: break;                           // Vector  8:  ADC12IFG1

case 10: break;                           // Vector 10:  ADC12IFG2

case 12: break;                           // Vector 12:  ADC12IFG3

case 14: break;                           // Vector 14:  ADC12IFG4

case 16: break;                           // Vector 16:  ADC12IFG5

case 18: break;                           // Vector 18:  ADC12IFG6

case 20: break;                           // Vector 20:  ADC12IFG7

case 22: break;                           // Vector 22:  ADC12IFG8

case 24: break;                           // Vector 24:  ADC12IFG9

case 26: break;                           // Vector 26:  ADC12IFG10

case 28: break;                           // Vector 28:  ADC12IFG11

case 30: break;                           // Vector 30:  ADC12IFG12

case 32: break;                           // Vector 32:  ADC12IFG13

case 34: break;                           // Vector 34:  ADC12IFG14

default: break;

}

}