A) Testing of PCB

- Connect PCB (with wires) to a fully charged Test Battery in proper sequence.

- Open GUI.  Import corresponding DF file.

- Connect Power supply (for few seconds to activate IC).

- If state of charge indication is not proper (i.e if <90%)  then give RESET command ( 0041)

- Manually Switch on the MOSFETs (by command 46 with word 0006)

- Perform following electrical tests:

    - Overcharge (COV)

    - Overcurrent in Discharge ( OCD1 & OCD2)

    - AFE Overcurrent Discharge protection ( AOCD)

    - Short circuit (SCD)

    - Reverse polarity (AOCD & SCD)

We have to Switch on the MOSFETs manually (by command 46 with word 0006) after every test.

- Remove fully charged battery and connect PCB (with wires) to a fully discharged Test battery.

- Connect Power supply (for few seconds to activate IC).

- If state of charge indication is not proper ( i.e if >20%) then give RESET command ( 0041)

- Perform following electrical tests.

  - Overdischarge (CUV)

  - Overcurrent in charge ( OCC1 & OCC2)

- Remove the PCB from Test battery.

B) Assembly of Actual battery

- Solder PCB on the actual cells ( Voltage >7.5V)  from most negative to higher positive.

- Check  for V3.3, V2.5!! If it  is not present, then connect to Charger / Power supply (with correct polarity 8.4V, 1A)

- Check for Test Points: V3.3; V2.5; VCC;

- If all the test points are showing proper voltages, then connect to EV2300

- Set Chemistry in GUI (  ERROR MESSAGE COMES..BUT CHEMISTRY UPDATES )

- Import flash constants DF file . Change Serial No., Mfg date, Lot code.

- Set Qmax = Design Capacity = Capacity of the same pack obtained earlier at C/5 or from data sheet!!

- Callibrate:

  - Stop scanning

  - Perform CC OFFSET Callibration

  - If still some offset current is present, Perform Software board calibration

  - Then go in sequence for calibration of Voltage then Temperature ( Current is not done)

- Give command 46 with word 0006 ( To switch on the FETs)

- Discharge the bty @ C/5 till 5.4V ( Term voltage)

- Rest for T>5 hrs

- Enable IT (SMB Command 00 ; SMB word: 0021).

- QEN & VOK in operation status should have been SET. ( CHG FET will automatically get ON)

- WAIT FOR 35MINUTES BEFORE CHARGING!!

- Charge fully until the taper current is reached & FC bit is set..

- Keep battery in rest for T>2 hours. (MaxError will become 5%.)

- Discharge the battery at C/5 rate till TERM VOLTAGE ( 6V).  (MaxError will become 5%)

- Keep battery in rest for 5hours. (MaxError will become 1% & UpdateStatus will become 6%_

- Repeat Charge & Discharge. Check for Operation Status bit R_DIS. It should be ‘0’. If it is ‘1’, our QMAX guess was wrong. Repeat it with the new QMAX obtained.

- Now seal the lid on the battery with ultrasonic sealing.

- Test the battery for capacity test and High rate discharge test.