# Startup

The output voltage and current at startup are shown in the image below. Input voltage is 230Vac. The output was loaded with a constant voltage load, set to 20Vdc, and a 5 Ohm resistor in series with the load. The measured current was 1.4A and the total voltage 27.5V.

Channel 2 shows the output voltage (5 V/div, 40ms/div).

Channel 4 shows the output current (500 mA/div).

# Efficiency

The efficiency data are shown in the tables and graph below. The power source is a 2KW California Instruments AC generator set to 180Vac, 230Vac and 264Vac. The current was set to 1.4A and a variable output power performed by modifying the output constant voltage load.

| Iout (mA) | Vout (V) | Pout (W) | Pin(W) | Vin (Vac) | PF (%) | Ploss (W) | Eff (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1443 | 11.99 | 17.30 | 20.446 | 180 | 94.63 | 3.14 | 84.62 |
| 1435 | 16.02 | 22.99 | 26.528 | 180 | 95.75 | 3.54 | 86.66 |
| 1435 | 20.00 | 28.70 | 32.670 | 180 | 96.43 | 3.97 | 87.85 |
| 1432 | 23.98 | 34.34 | 38.796 | 180 | 96.97 | 4.46 | 88.51 |
| 1430 | 27.51 | 39.34 | 44.350 | 180 | 97.28 | 5.01 | 88.70 |

| Iout (mA) | Vout (V) | Pout (W) | Pin(W) | Vin (Vac) | PF (%) | Ploss (W) | Eff (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1440 | 11.96 | 17.22 | 20.620 | 230 | 92.85 | 3.40 | 83.52 |
| 1437 | 16.02 | 23.02 | 26.757 | 230 | 94.69 | 3.74 | 86.04 |
| 1436 | 19.99 | 28.71 | 32.848 | 230 | 95.75 | 4.14 | 87.39 |
| 1433 | 23.99 | 34.38 | 38.993 | 230 | 96.54 | 4.62 | 88.16 |
| 1430 | 27.51 | 39.34 | 44.440 | 230 | 96.98 | 5.10 | 88.52 |

| Iout (mA) | Vout (V) | Pout (W) | Pin(W) | Vin (Vac) | PF (%) | Ploss (W) | Eff (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1439 | 11.97 | 17.22 | 20.801 | 264 | 90.46 | 3.58 | 82.81 |
| 1438 | 16.03 | 23.05 | 27.018 | 264 | 93.37 | 3.97 | 85.32 |
| 1436 | 19.99 | 28.71 | 33.073 | 264 | 95.10 | 4.37 | 86.79 |
| 1435 | 24.00 | 34.44 | 39.280 | 264 | 96.05 | 4.84 | 87.68 |
| 1431 | 27.52 | 39.38 | 44.668 | 264 | 96.63 | 5.29 | 88.16 |

# Output Current Regulation

The output current versus output power graph is plotted below.

# Power Factor

The Power Factor graph for the three input voltages is shown below:

# Output Ripple Voltage

The output ripple voltage is shown in the plot below. The input was set at 230Vac and the load was set to 1.4A, 27.5Vdc. Channel 1 shows the output ac voltage (0.5 V/div, 4ms/div).

# Output Ripple Current

The output ripple current is shown in the plot below. The input was set to 230Vac and the load was set to 1.4A, 27.5Vdc. Channel 1 shows the output current (0.2 A/div, 4ms/div).

# Switching Node Waveform

The image below shows the voltage on the drain of the switching node, with a 230Vac input, and a 1.4A, 27.5Vdc load.

# Input Voltage and Current Waveforms (same conditions)

# Loop Response

The image below shows the loop response of the converter measured with a 230Vdc input, and full load (1.4A, 27.5V, 38.5W). Phase margin is 76.79 deg. and crossover frequency is 34.46 Hz.