- Use TPS2384 EVM GUI with PSE device in Power Management Mode (PMM)

- Connect legacy PD (or simulate using parallel R-C) to the port. For this case, use a 10kohm resistor in parallel with a 10uF capacitor.

NOTE: The parallel resistance value will have some effect on the count value

- For the port under test (port 2 in this case), select the Capacitance function from the drop down menu, then click the appropriate port # to Write the function command.

- Select the Ramp Up function from the drop down menu and Write the command. Click the Read button to refresh the port readings.

- For this case, legacy volts running and volts legacy are displayed. Use the Voltage Raw value (4102 for this case) along with the chart in Figure 13 of the datasheet to find the capacitance value measured. For this case, 4102 counts correspond to ~12uF which is close to the actual value used.

- Test again using a 47uF capacitor.

1. Download the attached TPS2384EVM GUI Update (TPS2384 EVM 14-08-2006.exe)

2. Choose Power Management Mode. Follow the instructions in the pop up window.

3. Choose the Legacy function from the port drop down, then write the command.

4. Click the Read button and the capacitance voltages will be displayed.