Installing and using the bq20zXX BqMtester Program

Do not connect EV2300 until told to do so.

Discussion:  Open source code for bqMtester.

Install bqMTester Software:

- Copy  bqMTester Installation Software by unzipping the installation package into a directory with a short name (like c:\temp).

- Do not connect EV2300.

- Run unzipped installation EXE file.

- You may be asked to reboot your computer.  Select Yes, wait for reboot to complete and then run the installation again.

- Follow on screen instructions.   You may be asked to install EV2300 drivers if you have never installed any EV2300 compatible software on your system.  Just say yes and continue the installation.

- When done connect the EV2300 and follow on screen instructions to install the EV2300 drivers to your system.  There are 2 drivers that must be installed so the “Found New Hardware” popup will appear 2 times.   These are the selections to make for all the popup boxes:

  - No, not at this time

  - Install software automatically (Recommended)

  - Click Continue Anyway on the Windows Logo Testing screen.

  - It is common for the next screen to be the Confirm File Replace screen.  Click No to continue.  If this screen does not appear then go to the next step.

  - Remember there will be 2 drivers to install so the above screens will repeat 2 times.

- Discussion: Lets talk about the Targets file.

- Go to the X\:\Program Files\Texas Instruments\bq Tester\ directory and edit the file called “Targets” using notepad.

- Add the following line:

"bq20z90 v1.50","900.1.50"

To the bottom of the targets file.

- Save the “Targets” file

STOP!  Do not proceed until told to do so.

Golden Image File:

We will supply the golden image file required for this lab.

Connecting the supplied module and Cal Board:

There are 2 different Calibration boards (HPA169)  Follow one of the following pictures depending on your board layout.

Discussion:  Some people have seen issues with smbus passthrough

Figure 1.  New Revision Layout.  Use this if your board looks like above

Figure 2.  Prototype Revision Board (Connect B- to 1N of EVM)

Discussion:  If your board looks like Figure 2, the silkscreen was reversed on B- and P-.  A Sticker was placed on the board with the correct the error but just be sure to connect 1N to the 4th connector from the bottom regardless what the silkscreen says.

STOP!  Do not proceed until told to do so.

Setup:

- Select the appropriate Jumpers for the battery that you used.  We will be doing a 3 cell setup so set it up for 3 cells.  To Test 3 cells install a jumper at J7 labeled “1, 2, or 3 Cell” and another jumper at “3 Cell.  These are noted in Figure 1 or 2 by the small red circles..

- Run StationSetup.EXE

- Default password is bq20z80

- Search for boards and give the board a name

- Click “Save and Proceed”

- Select “Internal Probe” and then “Next”

Discussion:  Discuss the “internal probe vs external”.  How they can be configured.

- This section is the “Global Configuration” screen

- Here are the specifications for the bq20z90 board you are testing.  You find where they are relevant on the configuration screen and set it up accordingly.

  - 10 milliohm sense resistor

  - External Temp Sensor1

  - We will be calibrating CC Offset, Temperature, Voltage, and Pack Current.

  - Device is bq20z90

  - Version is 1.50

  - We will be updating the Data Flash

  - Voltage Reference is 1225mV

  - This is a 3 cell application

Discussion:  Discuss the following from the config screen:

- Discuss Volt Reference

- Serial Number

- Discuss the different Error % and what they mean

- Also compare the different temp sensors and relate to the one on the bqMTester

- Set Max OffsetTemperature to 50, %Errors to 25%.

Discussion:  Explain why Temp Offset is so high.

- Do nothing with Advance Calibration Frame

- Select “FET Off”.

- Select “External Load”

- Click “Save Configuration and Close”

STOP!  Do not proceed until told to do so.

Run MultistationTester:

- Run MultistationTester.exe

- Go to “Configure VTI”

- Unlock and measure Voltage using DMM.

- For current assume 1800mA and do not try and measure it.  Because lets face it. This is a real cheesy meter!

- Click “Read Currently calibrated Temperatures and assume temperature is 23 degrees C

Discussion:  Discuss how this temp offset works

- Input Voltage and Current in this window.  Also Change the temperature to 23 as noted in step 5.

- Click “Update VTI and Close”

- Click Start

- It should pass the test.

Discussion:

  - Error codes are in the manual.

  - Discuss the different error codes in both windows.

  - Current failures commonly bat- pack- mixup

  - Discuss differences between ROM and DFI files

  - Internal temp offset is way off.

  - Discuss bqMTester 2.X