| Register-Variable Name | Variable Type | Table Or Figure | Page | Addr | Bit Field | Signal Processing Section | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| en_ck_loss | D | 14 | G | 0 | 8 | General | (1) enable clock loss detection |
| pwr_dwn_firA | C | 14 | G | 0 | 9 | General | (1) disables PFIR A |
| pwr_dwn_firB | C | 14 | G | 0 | 10 | General | (1) disables PFIR B |
| pwr_dwn_firC | C | 14 | G | 0 | 11 | General | (1) disables PFIR C |
| pwr_dwn_firD | C | 14 | G | 0 | 12 | General | (1) disables PFIR D |
| pwr_dwn_CICAB | C | 14 | G | 0 | 13 | General | (1) disables CIC A, B |
| pwr_dwn_CICCD | C | 14 | G | 0 | 14 | General | (1) disables CIC C,D |
| Master_reset | C, SU | 14 | G | 0 | 15 | General | (1) reset chip |
| soB_sync | U | 15 | G | 1 | 2.0 | General | Normally set to Oneshot (4) or Sync Source |
| One_shot | C, SU | 15 | G | 1 | 4..3 | General | Pulse DUC – start (3, 2, 3) |
| page | Spec | 16 | G | 2 | 7..0 | General | Used with addresses 16 to 31d, for specific address extension page# |
| En_A0 | C | 17 | G | 3 | 0 | General | Tristate enable A Output[] |
| En_AFS | C | 17 | G | 3 | 1 | General | Tristate enable A port AFS, ACK |
| En_B0 | C | 17 | G | 3 | 2 | General | Tristate enable B Output[] |
| En_BFS | C | 17 | G | 3 | 3 | General | Tristate enable B port BFS, BCK |
| En_C0 | C | 17 | G | 3 | 4 | General | Tristate enable C Output[] |
| En_CFS | C | 17 | G | 3 | 5 | General | Tristate enable C port CFS, CCK |
| En_D0 | C | 17 | G | 3 | 6 | General | Tristate enable D Output[] |
| En_DFS | C | 17 | G | 3 | 7 | General | Tristate enable D port DFS, DCK |
| En_soB | C | 17 | G | 3 | 8 | General, DUC | Tristate enable SyncOut, IFLG – note required to be ON for Interleaved IQ Output mode |
| ckp_A | C | 17 | G | 3 | 9 | General | Invert ACK |
| ckp_B | C | 17 | G | 3 | 10 | General | Invert BCK |
| ckp_C | C | 17 | G | 3 | 11 | General | Invert CCK |
| ckp_D | C | 17 | G | 3 | 12 | General | Invert DCK |
| general_timer_ sync | U | T68 | xA1 | X12 | 9..7 | General | Sync selection for starting general timer |
| Gen_timer | U | T67 | Xa1 | X11 |  | General | General Timer 16bit counter, can be used for timer sync or sync output |
| gen_timer_test_ | U | T68 | xA1 | X11 | 6 | General | Shortens test counter uses top 16bits, no 256 prescaler |
| Mode AB, CD -transmit | P | T7 |  |  |  | General | AB mode, can be different from CD mode – receive or transmit |
| syncmode | P | T7(*) |  |  |  | General | Establishes sync mode settings (partial) Used for general setup |
| splitiq | P | T7 |  |  |  | DUC- Transmit Input | This variable sets up the channel for real processing instead of interleaved IQ. This also changes the TDM structure from IQ to I only per channel. |
| pins | P | T7 |  |  |  | DUC- Transmit Input | Determines the number of interface pins used for the DUC input, Input is MSB justified |
| bits | P | T7 |  |  |  | DUC- Transmit Input | Bits, used to determine the number of bits of resolution shifted through the interface, MSB justified to 20 bits. The allowable bits are 4,8,12, 16, and 20. The Parallel IQ (on a port) is limited to 8. The bits and pins settings affect the number of SCK transfers needed for an IQ input. |
| Tinf_cmplx | P (chan) | T7 |  |  |  | DUC- Transmit Input | Indicates that the input mode has complex data, parallel IQ, interleaved IQ, or TDM IQ |
| Tinf_tdm | P | T7 |  |  |  | DUC- Transmit Input | Indicates the input mode is TDM IQ, port A input only. |
| Tinf_fs_dly | P (chan) | T7 |  |  |  | DUC- Transmit Input | Parameter used to indicate the number of SCKs that the Frame Strobe is output early, to interface with the customer system. |
| Cksum_sync _back | U |  |  |  |  | DUC- Transmit Input |  |
| Tinf_bits | C (chan) | T30 | X13 | X10 | 2..0 | DUC- Transmit Input | Number of nibbles ( 4bits) used for signal resolution |
| Tinf_pins | C (chan) | T30 | X13 | X10 | 5..3 | DUC- Transmit Input | Number of pins used in IO interface 4,8,16 encoded |
| Tinf_iqmux | C (chan) | T30 | X13 | X10 | 6 | DUC- Transmit Input | 1 for interleaved IQ, 0 for real inputs (splitIQ) |
| Tinf_pariq | C (chan) | T30 | X13 | X10 | 7 | DUC- Transmit Input | 1 for special 8bit I, 8bitQ parallel IQ mode |
| Tinf_src | U/C (chan) | T30 | X13 | X10 | 10..8 | DUC- Transmit Input | Special control can be used for normal use or to select LFSR or constant-tone for test |
| Tinf_xmt | C (chan) | T30 | X13 | X10 | 11 | DUC- Transmit Input | 1 for transmit mode |
| Fso_sel | C (chan) | T30 | X13 | X10 | 13.12 | DUC- Transmit Input | 1 for transmit usage for Frame Strobe |
| Tinf_first | C (chan) | T31 | X13 | X11 | 3..0 | DUC- Transmit Input | Delay in counts to load first word in TINF format |
| Tinf_second | C (chan) | T31 | X13 | X11 | 7..4 | DUC- Transmit Input | Delay in counts to load second word in TINF format |
| Sck_div | C (chan) | T31 | X13 | X11 | 11..8 | DUC- Transmit Input | Channel Clock divider period is (1+sck_div) * CK period |
| Tinf_fso_dly | C (chan) | T31 | X13 | X11 | 14..12 | DUC- Transmit Input | Frame Strobe Output timing adjustment count |
| Tinf_fs_cnt | C (chan) | T32 | X13 | X12 | 11..0 | DUC- Transmit Input | Frame Strobe modulus in divided clocks |
| Tinf_nfs | C (chan) | T32 | X13 | X12 | 15..12 | DUC- Transmit Input | Number of Frame Strobes between FIR_Swap change |
| sck_sync | C/U (chan) | T51 | X14 | X15 | 14..12 | DUC- Transmit Input | Sync selection for sck_divider counter, should match the cic_sync and fir_sync selection if used |
| Tx_pat_gen | C/U | T44 | X13 | X1e | 14..13 | DUC- Transmit Input | Test input select for DUC, 2 for constant (tone output test), 1 for noise generator (spectral shape test) |
| overall_gain | P (chan) | T7(*) |  |  |  | DUC | Performs DUC calculations per channel, and calculates the gain_lsb,gain_msb,fir_shift, cic_shift, sum_shift |
| gain | P | T7 |  |  |  | DUC– Gain, AGC | Variable used to set the gain_lsb, gain_msb; value is relative to 1.0 for gain of 4096, not used with overall_gain command |
| gain_sync | U (chan) | T51 | X14 | X15 | 8..6 | DUC– Gain, AGC | Used to synchronize the gain register to manual gain update. Normally 6 or 7 for DUC |
| gain_lsb | C (chan) | T33 | X13 | X13 |  | DUC– Gain, AGC | Lower 16bits of 19bit gain word. This is the manual gain. |
| gain_msb | C (chan) | T34 | X13 | X14 | 2..0 | DUC– Gain, AGC | Upper 3 bits of 19bit gain word. This is the manual gain. |
| agc_freeze | U (chan) | T34 | X13 | X14 | 3 | DUC– Gain, AGC | 1 for Transmit |
| gain_rnd | C (chan) | T34 | X13 | X14 | 8..4 | DUC– Gain, AGC | Sets rounding for input to PFIR to 18 bits |
| fir_coef | P | T7 |  |  |  | DUC–PFIR | Pseudocommand to accept PFIR tap filename, based on the symmetry, PFIR configuration, loads the taps into the 16 FIR cells |
| Coefficient values | C (chan) |  | Page (0-f) 20,2f 40.4f 60.6f Adr (10-1f) | Page (0-f) 20,2f 40.4f 60.6f Adr (10-1f) | All | DUC–PFIR | PFIR Coefficients by channel, and coefficient number C0(first) – C255 Also sets up PFIR address generators, SwapRAM, BackEndRAM |
| fir_int | P (chan) | T7 |  |  |  | DUC–PFIR | PFIR interpolation amount, provides setup information for the PFIR coefficients separated into the 16 FIR cells, and input for the address generator setup |
| fir_diff | P (chan) | T7 |  |  |  | DUC–PFIR | PFIR different coefficients for I and Q (1) – not used |
| fir_nchan | P (chan) | T7 |  |  |  | DUC–PFIR | PFIR number of data channels in filter (1) – not used |
| fir_sync | U (chan) | T51 | X14 | X15 | 2..0 | DUC–PFIR | Provides synchronization selection for the FIR Controller, normally cic_sync, pfir_sync are the same source |
| coef_sync | U (chan) | T51 | X14 | X15 | 5..3 | DUC–PFIR | Provides coefficient address generator sync selection for the FIR controller, this is normally used at startup, but can be used to dynamically swap between two sets of coefficients, through the address generator offset controls |
| bypass_fir | P (chan) | T7 |  |  |  | DUC–PFIR | causes the system to use a decimate by 1, 16tap asymmetric PFIR, impulse |
|  | C (chan) | None | X10 | All |  | DUC–PFIR | Swap RAM pointer memory – used to re-order the IQ input data from the CIC, currently this is mapped as a bypass |
|  | C (chan) | T18 | X11 | All |  | DUC–PFIR | Backward End cell Control RAM – controls the backward end RAM delay, and blank feedback for end cell for odd symmetric filters |
| coef_ofsc | C | T19 | x12 | x10 | 3..0 | DUC–PFIR | FIR coefficient memory starting address |
| coef_modc | C | T19 | x12 | x10 | 7..4 | DUC–PFIR | FIR coefficient modulus count, start to end |
| coef_repc | C | T19 | X12 | x10 | 11..8 | DUC–PFIR | FIR coefficient address generator control, indicates the number of times the same coefficient is used |
| coef_sym | C | T19 | x12 | x10 | 12 | DUC–PFIR | FIR cell preadd forward and reverse delay lines |
| coef_zerofr | C | T19 | x12 | x10 | 13 | DUC–PFIR | FIR coefficient zero forward read – control is used for interpolation filter, for zero stuffing |
| agen_recr | C | T20 | x12 | x11 | 3..0 | DUC–PFIR | FIR forward delay line recirculation count – number of times the delay line data is reused |
| agen_depd | C | T20 | x12 | x11 | 7..4 | DUC–PFIR | FIR forward delay line depth count – number of delay line elements used |
| agen_modd | C | T20 | x12 | x11 | 11..8 | DUC–PFIR | FIR forward delay line cycle offset – this control is used if multiple signal sets are multiplexed through the filter |
| agen_togen | C | T20 | x12 | x11 | 12 | DUC–PFIR | FIR enable reverse delay – symmetric filter |
| fragen_soff | C | T21 | x12 | x12 | 3..0 | DUC–PFIR | FIR forward delay line read address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| fragen_srecr | C | T21 | x12 | x12 | 7..4 | DUC–PFIR | FIR forward delay line read address – recic counter – starting(sync) recirculation count |
| fragen_soff | C | T21 | x12 | x12 | 11..8 | DUC–PFIR | FIR forward delay line read address – depth counter – starting(sync) depth count |
| fragen_stepn | C | T21 | x12 | x12 | 15.. 12 | DUC–PFIR | FIR forward delay line read address –sample offset starting(sync) step size, indicates the address that the next block of signals will be read from |
| fwagen_soff | C | T22 | x12 | x13 | 3..0 | DUC–PFIR | FIR forward delay line write address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| fwagen_srecr | C | T22 | x12 | x13 | 7..4 | DUC–PFIR | FIR forward delay line write address – recic counter – starting(sync) recirculation count |
| fwagen_soff | C | T22 | x12 | x13 | 11..8 | DUC–PFIR | FIR forward delay line write address – depth counter – starting(sync) depth count |
| fwagen_stepn | C | T22 | x12 | x13 | 15.. 12 | DUC–PFIR | FIR forward delay line write address –sample offset starting(sync) step size, indicates the address that the next block of signals will be written to |
| bragen_soff | C | T23 | x12 | x14 | 3..0 | DUC–PFIR | FIR reverse delay line read address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| bragen_srecr | C | T23 | x12 | x14 | 7..4 | DUC–PFIR | FIR reverse delay line read address – recic counter – starting(sync) recirculation count |
| bragen_soff | C | T23 | x12 | x14 | 11..8 | DUC–PFIR | FIR reversedelay line read address – depth counter – starting(sync) depth count |
| bragen_stepn | C | T23 | x12 | x14 | 15.. 12 | DUC–PFIR | FIR reverse delay line read address –sample offset starting(sync) step size, indicates the address that the next block of signals will be read from |
| bwagen_soff | C | T24 | x12 | x15 | 3..0 | DUC–PFIR | FIR reverse delay line write address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| bwagen_srecr | C | T24 | x12 | x15 | 7..4 | DUC–PFIR | FIR reverse delay line write address – recic counter – starting(sync) recirculation count |
| bwagen_soff | C | T24 | x12 | x15 | 11..8 | DUC–PFIR | FIR reverse delay line write address – depth counter – starting(sync) depth count |
| bwagen_stepn | C | T24 | x12 | x15 | 15.. 12 | DUC–PFIR | FIR reverse delay line write address –sample offset starting(sync) step size, indicates the address that the next block of signals will be written to |
| beragen_soff | C | T25 | x12 | x16 | 3..0 | DUC–PFIR | FIR cell 16 read address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| beragen_srecr | C | T25 | x12 | x16 | 7..4 | DUC–PFIR | FIR cell 16 read address – recic counter – starting(sync) recirculation count |
| beragen_soff | C | T25 | x12 | x16 | 11..8 | DUC–PFIR | FIR cell 16 read address – depth counter – starting(sync) depth count |
| beragen_stepn | C | T25 | x12 | x16 | 15.. 12 | DUC–PFIR | FIR cell 16 read address –sample offset starting(sync) step size, indicates the address that the next block of signals will be read from |
| bewagen_soff | C | T26 | x12 | x17 | 3..0 | DUC–PFIR | FIR cell 16 write address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| bewagen_srecr | C | T26 | x12 | x17 | 7..4 | DUC–PFIR | FIR cell 16 write address – recic counter – starting(sync) recirculation count |
| bewagen_soff | C | T26 | x12 | x17 | 11..8 | DUC–PFIR | FIR cell 16 write address – depth counter – starting(sync) depth count |
| bewagen_stepn | C | T26 | x12 | x17 | 15.. 12 | DUC–PFIR | FIR cell 16 line write address –sample offset starting(sync) step size, indicates the address that the next block of signals will be written to |
| fwa_strobe |  | T27 | x12 | x18 |  | DUC–PFIR | 16 element times to write to next forward FIR cell |
| bwa_strobe |  | T28 | x12 | x19 |  | DUC–PFIR | 16 element times to write to next reverse FIR cell |
| Beagen_rbypass |  | T29 | x12 | x1a |  | DUC–PFIR | 16 element times to indicate Backend RAM bypass delay |
| Swap_wa_cnt |  | T46 | x14 | x10 | 3..0 | DUC–PFIR | FIR Swap RAM Write address count |
| Swap_ra_cnt |  | T46 | x14 | x10 | 7..4 | DUC–PFIR | FIR Swap RAM Read address count |
| Swap_xmt |  | T46 | x14 | x10 | 8 | DUC–PFIR | Swap RAM mode Transmit or receive |
| Swap_cmplx |  | T46 | x14 | x10 | 9 | DUC–PFIR | Swap RAM data is interleaved complex or Real |
| Fir_fb |  | T46 | x14 | x10 | 10 | DUC–PFIR | FIR forward broadcast |
| Swap_fb |  | T46 | x14 | x10 | 11 | DUC–PFIR | SwapRAM forward broadcast |
| Swap_rcv_tdly |  | T46 | x14 | x10 | 12 | DUC–PFIR | FIR swap RAM delay control for cic bypass |
| Acc_dly |  | T47 | x14 | x11 | 3..0 | DUC–PFIR | PFIR_Accumulator delay |
| Acc_cnt |  | T47 | x14 | x11 | 7..4 | DUC–PFIR | PFIR Accumulator, number of accumulations per cycle |
| Acc_dly0 |  | T47 | x14 | x11 | 8 | DUC–PFIR | FIR accumulator cycle delay |
| Acc_bypass |  | T47 | x14 | x11 | 9 | DUC–PFIR | Accumulator bypass – no partial accumulation |
| Acc_enram |  | T47 | x14 | x11 | 10 | DUC–PFIR | Enable partial accumulation RAM, in feedback path |
| fir_shift | C (chan) | T48 | X14 | X12 | 2..0 | DUC–PFIR | PFIR accumulator output (42bits) is shifted to an 18bit output, Fir shift has a variable 8 bit shift, along with a fixed shift. |
| Fir_xenq | C (chan) | T48 | X14 | X12 | 3 | DUC–PFIR | FIR transmit enable Q |
| Fir_xeni | C (chan) | T48 | X14 | X12 | 4 | DUC–PFIR | FIR transmit enable I |
| Fir_half | C (chan) | T48 | X14 | X12 | 5 | DUC–PFIR | Set to 0 |
| Fir_rnd20 | C (chan) | T48 | X14 | X12 | 6 | DUC–PFIR | 0 for DUC |
| Fir_cmplx | C (chan) | T48 | X14 | X12 | 7 | DUC–PFIR | FIR outputs complex interleaved data |
| Fir_sync_cnt | C (chan) | T49 | X14 | X13 |  | DUC-PFIR | Periodic FIR update counter |
| Fir_clk_cnt | C (chan) | T50 | X14 | X14 | 7..0 | DUC-PFIR | Number of FIR counts per CIC output - 1 |
| Fir_clk_cnt_ena | C (chan) | T50 | X14 | X14 | 8 | DUC-PFIR | 1 for normal, 0 for always |
| Fir_test | C (chan) | T50 | X14 | X14 | 9 | DUC-PFIR | 0 for normal |
| Fir_en_sync_cnt | C (chan) | T50 | X14 | X14 | 10 | DUC-PFIR | Enable fir sync counter |
| bypass_cic | P (chan) | T7 |  |  |  | DUC–CIC | Only in Split IQ mode, can be used to have a CIC_dec of 1 |
| cic_int | P (chan) | T7 |  |  |  | DUC–CIC | CICinterpolation amount |
| cic_shift | C (chan) | T52 | X80 | X19 | 5..0 | DUC–CIC | Used to scale the CIC input to the integrator, based on interpolation, and 5 or 6 stage CIC |
| cic_rcv | C (chan) | T52 | X80 | X19 | 7..6 | DUC–CIC | 2 for DUC |
| cic_xmt_5stg | C (chan) | T52 | X80 | X19 | 8 | DUC–CIC | ‘1’ for 5stage, ‘0’ for 6 stage |
| cic_2x | C (chan) | T52 | X80 | X19 | 10 | DUC–CIC | Reconfigures the I and Q dual CIC for even and odd CIC usage, enables additional logic between the 2 CICs |
| cic_xmt_d6stg | C (chan) | T52 | X80 | X19 | 11 | DUC–CIC | ‘1’ for 6stage or double rate CIC, ‘0’ else |
| cic_rcv_cross | C (chan) | T52 | X80 | X19 | 12 | DUC–CIC | 1 for split IQ |
| Cic_bypass | C (chan) | T52 | X80 | X19 | 14 | DUC–CIC | Used for CIC Bypass in Split IQ mode |
| flush_sync | C (chan) | F6, T58 | X80 | X16 | 14..12 | DUC–CIC | Sync selection to allow the CIC integrators to zero the feedback for initialization, or inserting zeros |
| ncic | C (chan) | T59 | X80 | X17 | 11..0 | DUC–CIC | CIC interpolator modulus (cic_int – 1) |
| cic_sync | U (chan) | F6,T59 | X80 | X17 | 14..12 | DUC–CIC | Sync Selection for cic_dec, to control the synchronization of the cic_decimation counter |
| freq | P (chan) | F6,T7 |  |  |  | DUC–Mixer | Calculates fractional frequency based on Fck, and freq |
| bypass_mix | P (chan) | T7 |  |  |  | DUC–Mixer | Causes mixer to be programmed for real input to I output, freq 0, phase 0, Gain 2^20-1 / 2^20 |
| Fck | P | F6,T7 |  |  |  | DUC–Mixer | Used with freq for fractional freq calcula |
| mix_icos | C (chan) | F5,T57 | x80 | x15 | 15..14 | DUC–Mixer | Select I for cosine multiply |
| mix_isin | C (chan) | F5,T57 | x80 | X15 | 12..11 | DUC–Mixer | Select I for sine multiply |
| mix_qcos | C (chan) | F5,T57 | x80 | x15 | 9..8 | DUC–Mixer | Select Q for cosine multiply |
| mix_qsin | C (chan) | F5,T57 | x80 | x15 | 6..5 | DUC–Mixer | Select Q for sine multiply |
| mix_inv_icos | C (chan) | F5,T57 | x80 | x15 | 13 | DUC–Mixer | Invert multiply result for subtraction |
| mix_inv_isin | C (chan) | F5,T57 | x80 | x15 | 10 | DUC–Mixer | Invert multiply result for subtraction |
| mix_inv_qcos | C (chan) | F5,T57 | x80 | x15 | 7 | DUC–Mixer | Invert multiply result for subtraction |
| mix_inv_qsin | C (chan) | F5,T57 | x80 | x15 | 4 | DUC–Mixer | Invert multiply result for subtraction, normally 1 |
| phase | U (chan) | F6, T53 | X80 | X11 |  | DUC–Mixer | 0 – 2^16 – 1 is 0 to 2Pi |
| freq_msb | C (chan) | F6, T56 | X80 | X14 |  | DUC–Mixer | Upper 15 bits and sign of 48bit delta phase value |
| freq_mid | C (chan) | F6 | X80 | X13 |  | DUC–Mixer | Middle 16bits of 48bit delta phase value |
| freq_lsb | C (chan) | F6 | X80 | X12 |  | DUC–Mixer | Lower 16bits of 48bit delta phase value |
| freq_sync | C (chan) | F6,T58 | X80 | X16 | 2..0 | DUC–Mixer | Select synchronization for control bus register deltaphase value to update input to phase accumulator |
| phase_sync | C (chan) | F6,T58 | X80 | X16 | 5..3 | DUC–Mixer | Select synchronization for control bus register phase offset value to update dither adder |
| dither_sync | C (chan) | F6, T58 | X80 | X16 | 8..6 | DUC–Mixer | Select synchronization for control bus to restart LFSR sequence for dithered phase |
| nco_sync | C (chan) | F6, T58 | X80 | X16 | 11..9 | DUC–Mixer | Select synchronization for control bus to reset phase accumulator |
| dither_test | C (chan) | T58 | X80 | X16 | 15 | DUC–Mixer | Changes the dither LFSR sequence |
| Tout_cmplx | P | T7 |  |  |  | DUC – Transmit Output, and Mixer | ‘1’ for Interleaved Complex or Parallel Complex modes, ‘0’ for Rea;. In Real mode causes Mixer Q path to be 0, note changes signal to output ports for parallel IQ modes |
| Tout_nsig | P | T7 |  |  |  | DUC – Transmit Sum | Number of DUC output ports, determines usage of sum blocks, 4 for individual outputs, 2 for A+B, C+D, 1 for A+B+C+D, also affects output channels select |
| Tout_rate | P | T7 |  |  |  | DUC – Transmit Output | Configures output ports for interleaved IQ 0, full rate 1, or double rate 2 |
| Tout_rate | P | T7 |  |  |  | DUC – Transmit Output | Configures output ports for interleaved IQ 0, full rate 1, or double rate 2, port mapping is different for double rate mode |
| Tout_res | P | T7 |  |  |  | DUC– Sum & Transmit Output | 0 for 16bits or less, 1 for 22bits, affects number of output ports available |
| Tout_sumin | P | T7 |  |  |  | DUC– Sum & Transmit Output | C and D outputs can be used as sum input ports, changes the number of available output ports to <3 |
| Sum_selI | C | T60,T4 | X81 | X10 | 7..0 | DUC Sum Control | Controls the A and B sum modes for I |
| Sum_selQ | C | T60,T4 | X81 | X10 | 15..8 | DUC Sum Control | Controls the A and B sum modes for Q |
| Sum_shift | C | T61 | X81 | X11 | 2..0 | DUC Sum Control | Sum output scaling for A and B summed outputs Calculated with overall gain |
| Sum_ia | C | T61 | X81 | X11 | 4..3 | DUC Sum Control | IA Sum Control |
| Sum_ib | C | T61 | X81 | X11 | 6..5 | DUC Sum Control | IB Sum Control |
| Sum_qa | C | T61 | X81 | X11 | 8..7 | DUC Sum Control | QA Sum Control |
| Sum_qb | C | T61 | X81 | X11 | 9 | DUC Sum Control | QB Sum Control |
| Sum_in | C | T61 | X81 | X11 | 11.10 | DUC Sum Control | Sum Input Mode Control, normally 0 |
| Toutf_hold_sync | C/U | T69 | Xa1 | X13 | 2..0 | DUC Output Control | Sync select for output decimator, used with real or parallel IQ output modes, should match cic_sync |
| Toutf_do | C | T69 | Xa1 | X13 | 8..4 | DUC Output Control | Signal selection for D Output port |
| Toutf_co | C | T69 | Xa1 | X13 | 11.9 | DUC Output Control | Signal selection for C Output port |
| Toutf_bo | C | T69 | Xa1 | X13 | 15..12 | DUC Output Control | Signal selection for B Output port |
| Toutf_rnd_AB | C/U | T70 | Xa1 | X14 | 1.0 | DUC Output Control | AB port round control, normally 16bit |
| Toutf_offsetbin | C | T70 | Xa1 | X14 | 3 | DUC Output Control | ‘1’ for offset binary output, 0 for 2s complement |
| Toutf_halfcmplx_AB | C | T70 | Xa1 | X14 | 4 | DUC Output Control | ‘1’ for interleaved complex output on AB output ports |
| Toutf_hold | C | T70 | Xa1 | X14 | 7..5 | DUC Output Control | ‘1’ to decimate ALL output ports, by value + 1 output counts |
| Toutf_rnd_CD | C | T70 | Xa1 | X14 | 9..8 | DUC Output Control | CD port round control, normally 16bit |
| Toutf_quiet_CD | C | T70 | Xa1 | X14 | 10 | DUC Output Control | Disables CD output ports for lower power |
| Toutf_halfcmplx_CD | C | T70 | Xa1 | X14 | 11 | DUC Output Control | ‘1’ for interleaved complex output on CD output ports |
| Toutf_xmtCD | C | T70 | Xa1 | X14 | 12 | DUC Output Control | ‘1’ for DUC mode, ‘0’ for DDC mode |
| Toutf_sumin | C | T70 | Xa1 | X14 | 13 | DUC Output Control | ‘1’ for sum input ports |
| Toutf_xmtAB | C | T70 | Xa1 | X14 | 14 | DUC Output Control | ‘1’ for DUC mode, ‘0’ for DDC mode |

Notes: (chan) – indicates that the variable is repeated on 4 different pages if the variable is individually controlled.  Variables that are shared between AB or CD have separate bit controls, and two addresses.  See Table 8 in the data sheet for the mapping of channels to tables.  The table indicates the common addresses and Channel A.

Legend

P – Pseudocommand,  SU – Startup Controlled, manual sequence

C – computed,	Spec – Page register is written each time that a different page of registers is accessed.

U – user provided