User Guide for Programming the

Texas Instruments GC5016

Digital Down Converter and Up Converter Device

Using the CMD5016 Software

Joe Quintal

Application Engineer

Wireless Radio Products Group

Texas Instruments

| Section# | Page# | Description |
| --- | --- | --- |
| 1.0 | 3 | CMD5016 General Information |
| 1.1 | 3 | Input Source Files |
| 1.2 | 4 | Comment Lines |
| 1.3 | 4 | Pseudocommands and Default values |
| 1.4 | 4 | Output File Options |
| 1.5 | 5 | GC5016 Mode Options |
| 1.6 | 5 | Combining Channels - SplitIQ |
| 1.7 | 6 | General Pseudo-Commands |
| 1.8 | 6 | Channel Identification and Copy-Channel Command |
| 1.9 | 6 | Channel Specific Commands |
| 1.10 | 6 | Example Index |
| 1.11 | 7 | Reference Documents |
| 2.0 | 7 | GC5016 DDC (Receive) commands and programming |
| 2.1 | 10 | GC5016 DDC - Input |
| 2.2 | 11 | GC5016 DDC - Mixer |
| 2.3 | 11 | GC5016 DDC - Decimating CIC filter |
| 2.4 | 11 | GC5016 DDC - Decimating PFIR filter |
| 2.5 | 12 | GC5016 DDC - Narrowband Power Meter and Manual or AGC Gain |
| 2.6 | 13 | GC5016 DDC – Output |
| 3.0 | 14 | GC5016 DUC (Transmit) Commands and Programming |
| 3.1 | 16 | GC5016 DUC - Input |
| 3.2 | 17 | GC5016 DUC - Narrowband Gain |
| 3.3 | 18 | GC5016 DUC - PFIR Interpolating Filter |
| 3.4 | 18 | GC5016 DUC - CIC Interpolation Filter |
| 3.5 | 18 | GC5016 DUC - Complex Mixer |
| 3.6 | 19 | GC5016 DUC – Output |
| Figure1 | 22 | CMD5016 Input Source Files |
| Figure2 | 23 | DDC GC5016 Application Diagram |
| Figure3 | 23 | DUC GC5016 Application Diagram |
| Figure4 | 24 | Transceiver GC5016 Application Diagram |
| Figure 5A-G | 25 | GC5016 DDC Block Diagram With Related Configuration Variables |
| Figure 6A-G | 32 | GC5016 DUC Block Diagram With Related Configuration Variables |
| Figure 7 | 39 | GC5016 DUC Sum Input Block Diagram |

1.0 CMD5016 General Information

The CMD5016 program is a DOS-Window or Linux compiled program used to configure the Texas Instruments GC5016 Digital Down Converter (DDC) / Digital Up Converter (DUC) device.  The software program interprets the user source files as a programming language does, selects the PFIR programming mode from working examples, and generates a programming file that can be used with customer software or EVM software to program the GC5016 device registers.

The CMD5016 program has two different sets of  programming inputs: pseudo-commands, and register field values.  The pseudo-commands are listed in Table 7 of the GC5016 datasheet.  The register field values are listed in Tables 8 through 71 of the GC5016 datasheet.  The user also specifies the FIR filter tap coefficient values in a signed decimal, single column format.

Sections 17.2 and 17.3 of the datasheet list the base set of commands that are needed for the DDC and DUC programming.  There are several examples later in this guide.

The CMD5016 output has several different output files that can provide register programming (.h or .gc101), and provide additional information on the setting of the registers based on the pseudo-commands and register field values selected (.anl, .dbg, and .tbl).

The GC101 Evaluation Module (EVM) software uses the CMD5016 DOS terminal application.  The GC101 evaluation platform software takes the source file or source information, and calls CMD5016.  The GC101 EVM software then loads the ‘name.gc101’ file-of-register-values into the GC5016.  The output from CMD5016 can also be used with modification for customer programming of the GC5016.

The CMD5016 program is invoked by typing, “cmd5016 ‘in_filename’”, where the in_filename is the cmd5016 source path and filename.

Figure 1 shows the source files used by the CMD5016 software and the generated output file.

Commands are identified in this document with italics.

1.1 Input Source Files

There are several categories of commands that are used with the CMD5016 program:

- Pseudo-commands that setup the (print) output files

- Pseudo-commands that setup the GC5016 operating mode

- Channel commands – there are 4 GC5016 channels that contain pseudo-commands and register field values.

- Programmable Finite Impulse Response Filter (PFIR) coefficient file.

The input file is a text only file.  No hidden or extended ASCII characters are accepted.

The fir coefficient file is a text only file.

1.2 Comment Lines

The input file has a general comment delimiter ‘#’.  All lines that start with ‘#’ are ignored by the interpreter software.

1.3 Pseudocommands and Default Values

Currently the data sheet pseudo-commands (Table 7), and  register fields (Tables 8 through 71) can have mandatory, default, and calculated values.  The suggested method is to select from the examples and modify a working configuration.

Some pseudo-commands and registers have default values, while some require user specification.  The .ANL file lists warnings and errors.   The .TBL output file lists the user-file-inputs at the bottom and the register field (calculated, default, and user) values at the top.

1.4 Output File Options

There are five output file options for CMD5016.  The keywords used to activate output file options are listed below:

print table – this prints a file with extension TBL, which when used with the data sheet

description lists all of the programmable fields for the GC5016

print analysis – this prints a file with extension ANL, which identifies the PFIR filter

configuration, specific error and warning messages, has gain details, and estimates power

consumption.

print debug – this prints a file with extension DBG, which identifies more details about

the gc5016 decimation and interpolation filter configuration.

print gc101 – this prints a file that is formatted for the GC101 evaluation platform.  The file is a

text file that contains the address and data values written to each register.

print hfile – this prints an insert file, which can be read by a C program, to configure the

GC5016 for an embedded application.  The C function reads the .h file, and with a customer

iowrite function, can program the GC5016.  The three element array structure is:

Page address, Register address, Register data.

The C program example is provided as config_gc5016.cpp..

1.5 GC5016 Mode Options

The GC5016 is unique in that there are two pairs of channels that can be configured in two modes; either as a DDC(receive) or DUC(transmit).  Figure 2 shows a DDC application.  Figure 3 shows a DUC application.  Figure 4 shows the combination as a transceiver application.  The mode command selects this capability.

mode AB receive – channels AB are in the DDC mode

mode AB transmit – channels AB are in the DUC mode

mode CD receive – channels CD are in the DDC mode

mode CD transmit – channels CD are in the DUC mode

DDC Example (See Figure 2):

mode AB receive

mode CD receive

DUC Example (See Figure 3):

mode AB transmit

mode CD transmit

Transceiver Example (See Figure 4):

mode AB transmit

mode CD receive

1.6 Combining Channels - SplitIQ

The GC5016 has either a two stage or single stage signal processing filter structure.  If the device is used in a standard 4 channel configuration, each channel’s filter strucuture calculates both the I and Q components.  This reduces the available number of FIR filtering taps.  In the splitIQ mode, the Programmable FIRs (PFIRs) in two channels can be combined to process one complex channel, increasing the available number of FIR filtering taps.

Note: Although there are more PFIR taps available, the choice of channel decimation (receive)

or interpolation (transmit) and internal storage may limit the number of PFIR coefficient taps.

The .ANL analysis file lists the available number of PFIR taps for each mode.

Note: The default for splitiq, splitiqAB, and splitiqCD is 0.

Splitiq 1 – this command indicates that all channels of the GC5016 will be placed in Split

IQ mode.

splitiqAB 1 – this command indicates that channels AB are in the split IQ mode

splitiqCD 1 – this command indicates that channels CD are in the split IQ mode

1.7 General Pseudo-Commands

Some commands affect the entire device.  These are listed above the channel section in the

input file.  They are:

fck – This is the rate, in Megahertz, of the GC5016 clock.   The clock rate is normally an

integer multiple of the symbol rate.   In the double rate mode, this is ½ the input or output rate.

sync_mode – This condition causes the CMD5016 program to program a specific startup

sync mode and sync selection.  The modes are selected based on user configuration.   A  single GC5016 configuration would use 9 for internally generated sync.  If the customer wants to generate a sync within the GC5016 and use it for the GC5016 and external logic, configurations 4 – 7 would be used.  If the GC5016 does not generate a sync, but responds to a sync on the SIA(A) or SIB(B) inputs, sync modes 0 – 3 would be used.  See sections 16.8 and 16.9 in the GC5016 datasheet for more detailed information.

# 0 - sync in A as slave

# 1 - sync in B as slave

# 2 - sync in A from external source

# 3 - sync in B from external source

# 5 - one shot output from soB syncing into sync A, pulse mode

# 7 - one shot output from soB syncing into sync B, pulse mode

# 9 - sync from one shot internally, pulse mode

soB_sync(*), gen_timer_sync(*), and gen_timer(*) are additional lower level commands.  See

the datasheet for specifics.

1.8 Channel Identification and Copying Channels

A common channel specific command is required as the first list in each channel programming section, “channel n”.

n = 0 for channel A,

n = 1 for channel B,

n = 2 for channel C,

n = 3 for channel D.

Another common channel command copies the settings from one channel to another.

copy_channel n is used to copy a previously defined channel.

1.9 Channel Specific Commands

The rest of the commands are placed under a specific channel.  These are listed in sections 2.0 (DDC-receive) and 3.0 (DUC-transmit).

1.10 Example Index

Examples are given below of different modes: tuning, filtering, and IO.  Select the one closest to your configuration as a starting point.  Each of the programming sections should follow the given channel command structure while selecting the proper programming values.

| Example Filename | Description |
| --- | --- |
| GC5016_DDC_4CH_INTIQ | 4 channel DDC – manual gain, interleaved IQ output |
| GC5016_DDC_4CH_AGC_TDMIQ | 4 channel DDC – manual & AGC gain, TDM output |
| GC5016_DDC_SPLITIQ_PARIQ | 2 channel SplitIQ DDC – manual gain, parallel IQ output |
| GC5016_DDC_DRIN_TDMIQ | 2 channel SplitIQ double rate real DDC – manual gain, TDM output |
| GC5016_DUC_4CH_8IQ_REAL | 4 channel DUC – 8bitI, 8bitQ parallel input, real output |
| GC5016_DUC_4CH_IntIQIn_IntIQOut | 4 channel DUC – 4 channel Interleaved IQ input, interleaved IQ output |
| GC5016_DUC_4CH_TDMIn_ParIQOut | 4 channel DUC – 4 channel TDM complex input, parallel complex output |
| GC5016_DUC_SplitIQ_ParIQIn_ParIQOut | 2 channel SplitIQ DUC – parallel complex input, parallel complex output |
| GC5016_2CH_Transceiver | 2 channel DDC, 2 channel DUC transceiver |
| GC5016_SplitIQ_1Ch_Transceiver. | 1 channel SplitIQ DDC, 1 channel SplitIQ DUC transceiver |

1.11 Reference Documents

GC5016 Data Sheet – Texas Instruments (SLWS 142F)

GC5016 Programming Tool for Standard Applications – Texas Instruments (SLWC043b)

GC5016 DDC Mode Gain by CMD5016 – Texas Instruments (SLWA032)

GC5016 DUC Mode Gain by CMD5016 – Texas Instruments (SLWA033)

GC5016 AGC Mode Settings by CMD5016 – Texas Instruments (SLWA031)

GC5016 AGC Application Note – Texas Instruments (SLWA027a)

GC5016 IO Application Note – Texas Instruments  (in review – not released)

DDC and DUC Fundamentals – Texas Instruments (FAE Guide)

2.0 GC5016 DDC (Receive) Commands and Programming

The DDC has several sections (See Figures 2 and 5A):

DDC Input – wideband input signals are received, the input channel is selected, and

additional rate conversion or interleaved data is processed.

Complex Mixer – The wideband input is combined with a complex mixer local oscillator

to generate a sum and difference complex output.

Cascade Integrator Comb (CIC) decimating filter – The complex mixer output is filtered

and decimated.  Scaling is applied to the data at the input, before the comb section, and at

the output.  In split IQ or double rate mode, there are inputs other than from the local mixer output.

Programmable FIR (PFIR) Decimating Filter – The filtered and decimated output from the CIC is further filtered and decimated in this stage.  Scaling is applied at the output of the filter.

Narrowband AGC, Power Meter, and Manual Gain - The filtered output has the power

level monitored and accumulated for a timer period.  The filtered output is adjusted for

manual gain or manual gain + AGC-error gain.

DDC Output – The I and Q output from the DDC can be output in several formats and

combinations of bit resolution and pins.

The overall_decimation is based on   (cic_dec * fir_dec) / zpad.

Although the order of programming the file is not important within a channel, following the signal processing order can help by describing the input to output sections in order.

| Variable name | Description | DDC Section |
| --- | --- | --- |
| mode AB receive mode CD receive | Determines DDC configuration |  |
| syncmode | Determines general settings for sync. See datasheet | DDC-sync |
| rin_rate | Receiver input rate, ½, full, or double rate input | DDC – Input, Mixer, CIC |
| rin_cmplx | Receiver input is real or complex | DDC – Input,Mixer |
| zpad(*) | Used to insert zeros for sub-rate digital input | DDC – Input,Mixer |
| mix_rcv_sel(*) | Selects one of the 4 input DDC busses | DDC – Input,Mixer |
| rinf_sel_’n’(*) | Low level mixer bus select, used only for diagnostic tests | DDC – Input,Mixer |
| rinf_diag(*) | Selects diagnostic source data | DDC – Input,Mixer |
| splitiq,splitiqAB, splitiqCD | Indicates two channels are combined for splitIQ | DDC-Mixer, CIC, PFIR |
| fck | Mixer DLO sample rate | DDC-Mixer |
| freq | Mixer DLO tuning frequency | DDC-Mixer |
| bypass_mixer | Identifies the mixer is bypassed, same as freq=0 | DDC-Mixer |
| phase(*) | Used in beamforming applications to set the offset phase | DDC –Mixer |
| cic_dec | Determines the CIC decimation factor | DDC-CIC, PFIR |
| bypass_cic | Identifies that the CIC is bypassed with cic_dec = 1. | DDC-CIC, PFIR |
| fir_diff | PFIR supports multiple data streams. Not currently supported. | DDC-PFIR |
| fir_nchan | PFIR number of data streams. Not currently supported. | DDC-PFIR |
| fir_dec | PFIR decimation factor. | DDC-PFIR |
| bypass_fir | Identifies fir_dec = 1, and impulse filter. | DDC-PFIR |
| fir_coef (filename) | PFIR signed decimal coefficients. | DDC- PFIR |
| pwr_mtr_on | Enables power meter | DDC-PowerMeter |
| pwr_mtr_integ(*) | Number of I or Q samples to integrate. | DDC Power Meter |
| overall_gain | Input to gain distribution algorithm, between CIC, FIR, and gain | DDC-CIC, PFIR, GAIN |
| gain | Sets gain_lsb and gain_msb as a ratio, superceded by overall_gain. | DDC- GAIN |
| gain_lsb(*), gain_msb(*) | Manual gain setting, where 4096 is 0db. See gain application note. | DDC- GAIN |
| gain_sync(*) | Selects the gain_sync source, which updates the manual gain value, and zeros the AGC_gainerror accumulator. | DDC-Gain, AGC |
| agc_freeze(*) | Holds the agc error accumulator value. It is set to 0 to start agc gain adaption. | DDC-Gain, AGC |
| agc_mode | Selects agc mode. See agc application note. | DDC-Gain, AGC |
| agc_tc | Sets AGC D parameters based on output sample rate, and desired agc time constant (us). | DDC-Gain, AGC |
| agc_cf | Determines the average target for the agc output, to allow for crest factor headroom (db). | DDC-Gain, AGC |
| bits | Determines the number of output bits that are rounded. Also used for muxed output formats. | DDC – Output |
| pins | Number of output pins used. | DDC –Output |
| routf_tdm | TDM format. All DDC channels are output on same port. | DDC –Output |
| routf_iqmux(*) | Interleaved IQ on DDC output channels. | DDC –Output |
| sck_div(*) | Controls output timing by dividing the output frame rate. | DDC –Output |

Note (*) denotes a register field as defined in tables 8 through 71.  All other variables are listed in table 7 in the datasheet.

2.1 GC5016 DDC - Input (See Figures 5A, 5B)

The GC5016 IO application note and the GC5016 datasheet have more details for this section.  The general programming items are listed here:

Real Input – Each input port represents a real sample, sampled with the Ck clock.

Interleaved Complex – Each input port represents a ½ rate complex signal.  The I or Q data is

received on each Ck clock.  A special synchronization from the sync input 0->1 to the Q input   is required for this mode .

Parallel Complex – A pair of inputs, A,B and C,D, are used to input the full rate complex

sample.  The I and Q data is received on each Ck clock.

Double Rate Real Input – A pair of inputs, A,B and C,D, are used to input double rate data as

even and odd samples.  The even and odd samples are received on each Ck clock.

Double Rate Complex Input – All four input ports are used to sample one double rate complex

signal.  The double rate complex input can be processed by DDC A,B and C,D separately.

| DDC Input Mode | rin_rate | rin_cmplx | Ports used |
| --- | --- | --- | --- |
| Real – Full Rate | 1 | 0 | 1 port, 2s complement Real input |
| Interleaved Complex – Half Rate | 0 | 1 | 1 port, 2s complement, Interleaved complex input, synchronized to zpad_sync |
| Parallel Complex – Full Rate | 1 | 1 | 2 ports, A-B or C-D. A&C are I, B&D are Q |
| Real – Double Rate | 2 | 0 | 2 ports, A-B or C-D. A&C are even real, B&D are odd real |
| Complex – Double Rate | 2 | 1 | 4 ports: A-even I, B-even Q, C-odd I, D-odd Q |

Zpad is used with a sub-rate digital input.  The most common use is with a ½ rate DDC.  This value applies to all DDC channel inputs.  The zpad(*), and zpad_sync(*) are register bit fields for programming this function.

Note: The rinf_sel_’n’(*), where n is the A,B,C, or D input bus, and the value is set to 8, selects the diagnostic input.  The rinf_diag(*) is typically set to 3.  This provides a x4000 DC input to the mixer, and can provide a complex tone at the mixer frequency at the output, for diagnostics.

2.2 GC5016 DDC - Complex Mixer (See Figures 5A, 5C)

The ‘mix_rcv_sel(*)’ command is used to select one of four input busses.  The value to select an input bus is:

0-A

1-B

2-C

3-D

The mixer accumulates the local oscillator phase information, and converts it to an inphase and quadrature value that is multiplied by the selected input data.  The freq and Ck are used to determine the phase_delta accumulated on each cycle.

Deltaphase = freq/fck * 2^48

There is an offset term added to the accumulated phase, controlled by the phase register.

Note: If the ADC sampling and input signal are in Nyquist zone 1or 3, a negative tuning frequency is used.  If the signal is in Nyquist zone 2 or 4, a positive tuning frequency is used for image tuning.

2.3 GC5016DDC -  Decimating CIC Filter (See Figures 5A, 5D)

The CIC filter is a 5 stage CIC that can decimate from 1 to 222 without special programming.

Setting the decimation uses the cic_dec command.

Note: The CIC decimation can be extended to higher values, depending on PFIR hardware symmetry and sign extended input modes.   The CIC can be bypassed in splitIQ mode, or if special real-channel fir_cmplx(*) only programming is used.

2.4 GC5016 DDC - Decimating PFIR Filter (See Figures 5A, 5E)

The PFIR is used differently in splitIQ mode versus the standard configuration.  The PFIR in splitIQ mode processes I and Q in separate (I-A,C; Q-B,D) channels.  The standard configuration has an  I and Q data stream with common coefficients in each PFIR channel.

Typically the PFIR is configured as a decimation factor and a filter coefficient (fixed point center tap scaled to 32767) file.

The PFIR normally decimates by 2 to 8 in integer values.  The PFIR in split IQ mode can decimate from 1 to 16 in integer values.

The PFIR decimation is programmed by setting fir_dec.

The PFIR coefficients are programmed in a file identified with the fir_coef command.

The fir_shift(*) scales the PFIR filter output.  Its value is programmed through the overall_gain value, or manually.  See the GC5016 DDC Gain application note or the datasheet for the calculation.

See the DDC and DUC Fundamentals section on PFIR coefficient design for a single and dual stage DUC.  The number of taps designed should match one of the .ANL file values for proper PFIR programming.

2.5 GC5016 DDC - Narrowband Power Meter and Manual or AGC Gain (See Figures 5A and 5F)

The narrowband power meter accumulates the I^2 and Q^2 values over a timer interval.  The accumulated value is read out over the control bus, with synchronization.

pwr_mtr_on 1 is used to enable the power meter accumulation for this channel.

pwr_mtr_integ - Number of I or Q samples to integrate.  The power meter integration period is the Ck_period * cic_dec * fir_dec * pwr_mtr_integ/2.

Note: In the split IQ mode, the A or C channels accumulate the I power meter data. The B or D channels accumulate the Q power meter data.

The narrowband gain can have manual only or manual and agc_error gain.  The gain_sync value selects the update of the manual gain registers, and clears the AGC gain_error accumulator.   The agc_freeze can be used to hold the AGC gain_error accumulator value.  The agc_freeze control can be used when the gain adaption is done only for a portion of the DDC output cycle.

| Narrowband Gain Mode | gain_sync | agc_freeze | selected external sync |
| --- | --- | --- | --- |
| Manual – asynchronous | 6,7 | 1 | NA |
| Manual – synchronous | 2,3 | 1 | (2) SIA pulse, (3) SIB pulse |
| Man+AGC – asynchronous | 0,1 | 0 | NA |
| Man+AGC-synchronous | 2,3 | 0 | (2) SIA pulse, (3) SIB pulse |

agc_mode – this control is used to develop the attack, decay, and threshold values, and sets specific model constants for the time constant and threshold below.

agc_tc – this control sets the D values for the attack and decay time in usec for the AGC settings.

agc_cf – this determines the threshold value based on the agc_mode signal statistics and the desired crest factor at the output.

Note: AGC operation in the split IQ mode can result in IQ imbalance and requires special considerations outside standard AGC programming.

Manual gain update can be done synchronously or asynchronously.  If asynchronous gain update is used, the user needs to write to the gain_lsb and gain_msb register-fields after programming.  The gain_sync can be set to the desired sync source.

Note: See the DDC Gain application note for setting overall_gain, or the manual gain gain_lsb, gain_msb, fir_shift.

2.6 GC5016 DDC - Output (See Figures 5A and 5G)

The DDC output modes are used to output real, interleaved complex, parallel complex, or time-division multiplexed complex data from the GC5016:

Real Output – in the special case where the DDC is used as a FIR filter, each

channel has a real parallel output.

Parallel Complex Output – in the split IQ mode, when two DDC channels are used together, the I and Q data can be output in parallel.  Each I or Q output is an A,C-I, or B,D-Q.  The output port programming has real A and B or real C and D.  However, the pair of ports is a complex output.

Interleaved Complex Output – each of the DDC outputs has separate pins which can be used to output interleaved IQ data for the connected channel.

Time Division Multiplexed (TDM) Complex Output – combines all the DDC channels at the output port.  All channels must be at a common decimation to use this feature.  The 4 channel output is DI, DQ, CI, CQ, BI, BQ, AI, and AQ.  The Split IQ output is DQ, CI, BQ, and AI.

| DDC Output Mode | routf_tdm | routf_iqmux(*) |
| --- | --- | --- |
| Real | 0 | 0 |
| Parallel complex | 0 | 0 |
| Interleaved Complex | 0 | 1 |
| TDM Complex | 1 | 0 |

The channel clock can be divided by the sck_div(*) register value.  The channel clock can be 1:1 to 1:16 based on the sck_div value of 0 to 15.   The typical value is pfir_dec or less.  In the TDM output modes, there are 4 or 8 subframes.

Note: There is a special mode that can interleave the gain value with the I and Q value at the DDC output.  agc_gain_out = 1, pins = 16, and bits = 8 are used for this special mode.  See section 13.12.3 in the datasheet.

Bits and pins are two other pseudo-commands used to specify the DDC output interface.  This allows the designer to trade off the number of IO pins used versus the speed of the data on these pins.  The bits can be 4,8,12,16 or 20.  The pins can be 4, 8, or 16.  The GC5016 IO application note has tables for each configuration. An example is provided below:

Parallel IQ output – 20bits, 4 pins. This takes 5 output clocks to send the IQ data over the 4-I, 4-Q, FS, and ChClk;  10 wires/ch; num channel clocks to output IQ = bits/pins.

Interleaved IQ output – 20bits, 4 pins. This takes 10 output clocks to send the IQ data over the 4-IQ, FS, and ChClk; 6 wires/ch;  num channel clocks to output IQ = 2 * bits/pins.

TDM 4 channel mode – 16bits, 8 pins. This takes 16 output clocks to send the 4 channels of IQ data over 8IQ, FS, and ChClk; 10 wires/4ch; num channel clocks to output IQ = 8*bits/pins

TDM 2 channel mode – 16bits, 8 pins. This takes 8 output clocks to send the 2 channels of IQ data over 8IQ, FS, and ChClk; 10wires/2ch; num channel clocks to output IQ = 4*bits/pins

3.0 GC5016 DUC (Transmit) Commands and Programming

The DUC has several sections (see Figure 5A), and several options in multiple device

implementation (See Figure 6).

The DUC sections :

DUC Input – the narrowband input is received and, if needed, demultiplexed.

If diagnostic mode is selected, a diagnostic value is added.   The input block receives a Next IQ signal from the PFIR that sends out the frame strobe request for the next IQ data frame.

Narrowband Gain – The selected input is multiplied by the real or complex input data.

In the 4 channel mode interleaved I and Q data is processed in each channel.  In the splitIQ mode, channels A and C process I data, and B and D process Q data.

Programmable FIR (PFIR) Interpolating Filter– The gain adjusted data is interpolated and filtered.  Scaling is applied at the output of the filter.

Cascade Integrator Comb (CIC) - PFIR output is filtered and interpolated.  Scaling is applied to the data at the input, before the integrator section, and at the output.  There are outputs that are used for other channels in the splitIQ or double rate modes.

Complex Mixer – The interpolated and filtered CIC data is mixed by the digital local oscillator.  In the split IQ or double rate mode the mixer inputs come from multiple channels.  The complex or real output goes to the sum shifted block.

SumBus and Output – The internal channels and,if enabled, the Cout and Dout inputs

(Sum Input bus) are combined. The SumA and SumB busses are scaled, and the user selected output is sent to the output ports.  Ports that are not used are usually tristated to reduce the IO power.

The overall_interpolation is based on   (fir_int*cic_int) / output_hold.

Although the order of programming the file is not important within a channel, following the signal processing order can help by describing the input to output sections in order.

| Variable name | Description | DUC Section |
| --- | --- | --- |
| mode AB transmit mode CD transmit | Determines DUC configuration. |  |
| syncmode | Determines general settings for sync. See datasheet. Note EVM setting is different. | DUC-sync |
| splitiq,splitiqAB, splitiqCD | Indicates two channels are combined for splitIQ. | DUC-Input, PFIR, CIC, Mixer |
| sck_div(*) | Sets the channel clock divider, determines the frame strobe width. | DUC-Input |
| tinf_fs_dly | Determines the timing advance for the frame strobe signal relative to the GC5016 DUC sample point. | DUC-Input |
| tinf_tdm | Used to indicate the time division multiplex mode for the A input port. | DUC-Input |
| tinf_iqmux(*) | Used to indicate the interleaved I and Q on each input port. | DUC-Input |
| tinf_cmplx | Used to indicate the DUC input data is complex. | DUC-Input |
| tinf_pariq(*) | Used with bits and pins to provide an 8bitsI and 8bitsQ in parallel. | DUC-Input |
| bits | Determines the number of input bits and scaling used for DUC input. | DUC – Input |
| pins | Number of input pins used for the DUC input port. | DUC – Input |
| overall_gain | Input to gain distribution algorithm, between output shift, cic, fir, and gain. | DUC- GAIN, PFIR, CIC, OUTPUT |
| gain_lsb(*), gain_msb(*) | Manual gain setting, where 4096 is 0db. See Gain application note. | DUC- GAIN |
| fir_diff | PFIR supports multiple data streams. Not currently supported. | DUC-PFIR |
| fir_nchan | PFIR number of data streams. Not currently supported. | DUC-PFIR |
| fir_int | PFIR interpolation factor. | DUC-PFIR |
| bypass_fir | Identifies fir_dec = 1, and impulse filter. | DUC-PFIR |
| fir_coef (filename) | PFIR signed decimal coefficients. | DUC- PFIR |
| cic_int | Determines the CIC interpolation factor. | DUC-CIC, PFIR |
| bypass_cic | Identifies that the CIC is bypassed with cic_dec = 1. | DUC-CIC, PFIR |
| fck | Mixer DLO sample rate. | DUC-Mixer |
| freq | Mixer DLO tuning frequency. | DUC-Mixer |
| bypass_mixer | Identifies the mixer is bypassed, same as freq=0. | DUC-Mixer |
| phase(*) | Used in beamforming applications to set the offset phase. | DUC –Mixer |
| tout_cmplx | Determines if the DUC output is real or complex. | DUC – Mixer, Output |
| tout_rate | Determines if the DUC output rate is ½, full, or double rate. | DUC –Output |
| tout_sumin | Determines if the COut and DOut ports are reassigned as sum input ports. | DUC –Output |
| tout_res | Determines if the AB and CD output ports are used as separate 16bit ports or combined for a 22bit port. | DUC –Output |
| toutf_hold(*) | Used to control output decimation.. | DUC –Output |
| tout_nsig | Controls the DUC channel summing, as A+B+C+D on the A port, A+B on A, C+D on B, or individual outputs. | DUC –Output |

3.1 GC5016 DUC - Input (See Figures 6A, 6B)

The DUC Input has frame strobe and channel clock output pins, and the input port.

The DUC input other than TDM is not mapped across other channels.  The DUC input can be operated in several modes. Some of the modes are dependent on the channel operating mode:

Real – If the DUC is used as an interpolating FIR filter, the DUC input corresponds to the FIR filter input.   The frame strobe signal and divided channel clock are used to request the next real input.   Each of the DUC channels can operate independently.

Parallel Complex (Split IQ) – If the DUC is used in the split IQ mode, the A,B, and C,D pairs of channels have I and Q inputs.  The frame strobe signal and divided channel clock are used to request the next complex input.    The A,B and C,D pairs of channels must have the same settings.  They don’t have to be the same interpolation.

Parallel Complex (8 bit) – Each of the DUC channels has an 8 bit I and 8 bit Q interface over the 16 pin interface.  The frame strobe signal and divided channel clock are used to request the next complex input.  Each of the DUC channels can operate independently.

Interleaved Complex IQ – Each of the DUC channels can operate with an interleaved I then Q interface over the channel DUC input.  The frame strobe signal and divided channel clock are used to request the next complex input.  Each of the DUC channels can operate independently.

TDM Complex – The A input port is used to transfer data synchronously to the active DUC input channels.  The TDM mode can support both split IQ and four channel mode.  The active DUC channels must be at a common interpolation rate for this to work properly.

The variables that control the DUC input mode are listed in the table below:

| Description | splitiq | tinf_tdm | tinf_cmplx | tinf_iqmux(*) | tinf_pariq(*) |
| --- | --- | --- | --- | --- | --- |
| 4 channel TDM | 0 | 1 | 1 | 0 | 0 |
| 4 channel 8bitsI 8bitsQ 16 pin | 0 | 0 | 1 | 0 | 1 |
| 4 channel interleaved IQ | 0 | 0 | 1 | 1 | 0 |
| 4 channel parallel IQ > 8 bit real | 0 | 0 | 0 | 0 | 0 |
| 2 channel parallel IQ > 8 bit real | 1 | 0 | 0 | 0 | 0 |
| 2 channel TDM (split IQ) | 1 | 1 | 0 | 0 | 0 |
| 2 channel parallel IQ > 8 bit parallel, parallel complex (split IQ) | 1 | 0 | 1 | 0 | 0 |

The sck_div(*) and tinf_fs_dly determine the frame strobe to data timing for the DUC input.

3.2 GC5016 DUC - Narrowband Gain (See Figures 6A, 6C)

The input I and Q data is scaled to 20 bits, and multiplied by the gain_lsb and gain_msb.

The gain adjusted output is scaled back to 18 bits for the PFIR.  The overall_gain includes the

narrowband gain adjustment,  and other gain adjustments.  See the DUC Gain application note

for the manual gain calculation.  The overall_gain can be set in general as 1/(number of channels).

Note: In the splitIQ mode, the Channel A,B or C,D gains must match to prevent IQ imbalance.

Note: If the PFIR tap configuration uses the hardware symmetry, the input gain must be limited

to .5.

3.3 GC5016 DUC - PFIR Interpolating Filter (See Figures 6A, 6D)

The PFIR is used differently in split IQ mode versus the standard configuration.  The PFIR in split IQ mode processes I and Q in separate (I in A,C; Q in B,D) channels.  The standard configuration has an I and Q data stream with common coefficients in each PFIR channel.

Typically the PFIR is configured as an interpolation factor and a filter coefficient (fixed point center tap scaled to 32767) file.

The PFIR normally interpolates by 2 to 8 in integer values.  The PFIR in split IQ mode can interpolate from 1 to 16 in integer values.  If the PFIR interpolates by 1 or 2, hardware symmetry can be used.  The PFIR interpolation is programmed by setting fir_int.

The PFIR coefficients are programmed in a file identified with the fir_coef pseudo-command.

The fir_shift(*) scales the PFIR filter output.  Its value is programmed through the overall_gain value, or manually.  See the GC5016 DUC Gain application note or the datasheet for the calculation.

See the DDC and DUC Fundamentals section on PFIR coefficient design for a single and dual stage DUC.  The number of taps designed should match one of the .ANL file values for proper PFIR programming.

3.4 GC5016 DUC - CIC Interpolation Filter (See Figures 6A, 6E)

The CIC filter is a 5 or 6 stage CIC that can interpolate from 1 to 256 without special programming.  In application, if the CIC interpolation is > 192, the 5 stage CIC is used.  Setting the CIC interpolation is done with the cic_int pseudo-command.

Note: The CIC can be bypassed in splitIQ mode or if special real-channel only programming is used.

3.5 GC5016 DUC - Complex Mixer (See Figures 6A, 6F)

The mixer in the DUC is configured based on the tout_cmplx, tout_rate, and splitiq commands.

The complex mixer inputs are selected based on the current channel or paired splitIQ cross channel.  This is configured internally through CMD5016.

If the tout_cmplx signal is a value of 0, the complex mixer is programmed for real output mode.

The mixer accumulates the local oscillator phase information, and converts this to the inphase and quadrature value that is multiplied by the selected input data.

The freq and Ck are used to determine the phase_delta accumulated on each cycle.

Deltaphase = freq/fck * 2^48

There is an offset term added to the accumulated phase, controlled by the phase register.

The mixer frequency is additive for the complex DUC output.

3.6 GC5016 DUC - Sum Busses and DUC Output (See Figures 6A and 6G)

Sum Busses

If the tout_sumin is a 1,the C and D putput ports are reprogrammes as sum inputs.

The sum busses internal to the GC5016 A and B can be individual or summed channels.  The tout_nsig indicates the number of channels that are added for a sum bus.   There are two sum busses in the GC5016 DUC output.   These are normally controlled by tout_nsig, and the number of DUC output channels.    The two sum busses have a manual gain shift sum_shift. This is normally adjusted using the overall_gain command.  The sum_shift(*) is used to scale the summed data to the output pins.  The overall_gain adjusts the sum_shift(*) value.    The C and D individual channel outputs have a fixed value for sum_shift.   See the GC5016 DUC Gain application note for more details.

DUC Output Mode Settings

The DUC Output is configured through several pseudo commands and several register variables.  A detailed description is found in the GC5016 datasheet and the GC5016 IO Application note.

Note: The output can have 12, 14, 16, or 22 bit rounding. The variables that control the DUC output rounding are toutf_rnd_AB, and toutf_rnd_CD.   The output rate can be less than full rate (decimated), full rate, interleaved complex, or double rate.  These are controlled through the toutf_cmplx and toutf_rate values.

The GC5016 has several DUC output modes.  They are individually described in the following sections.  See Mode table, page 20..

DUC Real Output

The DUC real output has 16 or 22 bit output modes.  The 16bit output mode utilizes one 16-pin output port.  The 22 bit output mode utilizes two output ports.   tout_res and tout_cmplx control these settings.

DUC Interleaved IQ Output

The four GC5016  output ports can be used with a ½ rate complex output.  In this case, the interpolation occurs from the symbol input to the clock rate and the output is decimated by 2 in the output logic.   The I and Q outputs are interleaved onto a single set of output pins. The IFLG signal identifies when the I signal of the A port is output.  The interleaved complex output can be 16 or 22 bits.  The toutf_res, toutf_cmplx, and toutf_rate controls setup the interleaved IQ output mode.

DUC Parallel Complex Output

The GC5016 can have two parallel full rate complex outputs with 16 bit resolution, or one output with 22bit resolution.    The I and Q are output on the I-A,C and Q-B,D ports.   Extended 22 bit resolution can occur by using two ports for the 22 bit outputs.   The toutf_res, toutf_cmplx, and toutf_rate controls setup the parallel complex IQ output mode.

DUC Double Rate Real Output

The DUC can have real outputs at double rate.  There are two outputs (odd and even) for each clock cycle.  The toutf_res, toutf_cmplx, and toutf_rate control these conditions.

DUC Parallel Complex Double Rate Output

The 16 bit parallel complex can also be used at double the Ck rate.  Port A,B provides the IQ even output, and port C,D provides the IQ odd output.  toutf_res, toutf_cmplx, and toutf_rate control these conditions.

Mode Table for Sum Bus and DUC Output

| Output type – sum input not used (tout_sumin = 0) | tout_ rate | tout_ res | tout_ nsig | tout_ cmplx |
| --- | --- | --- | --- | --- |
| Real output- full rate-16bits Separate outputs | 1 | 0 | 4 | 0 |
| Real output- full rate-16bits A+B->A,C+D->B | 1 | 0 | 2 | 0 |
| Real output- full rate-16bits A+B+C+D->A | 1 | 0 | 1 | 0 |
| Real output- full rate-22bits A+B->AB; C+D->CD | 1 | 1 | 2 | 0 |
| Real output- full rate-22bits A+B+C+D->AB | 1 | 1 | 1 | 0 |
| Real output double rate – 16bits; AB->AB;CD->CD | 2 | 0 | 2 | 0 |
| Real output double rate – 22bits; AB+CD->AB | 2 | 1 | 1 | 0 |
| Interleaved IQ – half rate – 16bits, Separate outputs | 0 | 0 | 4 | 1 |
| Interleaved IQ – half rate – 16bits, A+B->A, C+D->B | 0 | 0 | 2 | 1 |
| Interleaved IQ – half rate – 16bits, A+B+C+D->A | 0 | 0 | 1 | 1 |
| Interleaved IQ – half rate – 22 bits, A+B->AB;C+D->CD | 0 | 1 | 2 | 1 |
| Interleaved IQ – half rate – 22 bits, A+B+C+D->AB | 0 | 1 | 1 | 1 |
| ParallelIQ output- full rate-16bits AB->AB, CD->CD | 1 | 0 | 2 | 1 |
| ParallelIQ output- full rate-16bits A+C,B+D->AB | 1 | 0 | 1 | 1 |
| ParallelIQ output – full rate 22bits A+C,B+D->ABCD | 1 | 1 | 1 | 1 |
| ParallelIQ output – double rate 16bits AB+CD->ABCD | 2 | 0 | 1 | 1 |