| Register-Variable Name | Variable Type | Table Or Figure | Page | Addr | Bit Field | Signal Processing Section | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| en_ck_loss | D | 14 | G | 0 | 8 | General | (1) enable clock loss detection |
| pwr_dwn_firA | C | 14 | G | 0 | 9 | General | (1) disables PFIR A |
| pwr_dwn_firB | C | 14 | G | 0 | 10 | General | (1) disables PFIR B |
| pwr_dwn_firC | C | 14 | G | 0 | 11 | General | (1) disables PFIR C |
| pwr_dwn_firD | C | 14 | G | 0 | 12 | General | (1) disables PFIR D |
| pwr_dwn_CICAB | C | 14 | G | 0 | 13 | General | (1) disables CIC A, B |
| pwr_dwn_CICCD | C | 14 | G | 0 | 14 | General | (1) disables CIC C,D |
| Master_reset | C, SU | 14 | G | 0 | 15 | General | (1) reset chip |
| soB_sync | U | 15 | G | 1 | 2.0 | General | Normally set to Oneshot (4) or Sync Source |
| One_shot | C, SU | 15 | G | 1 | 4..3 | General | Level DDC – start (1, 0, 1) Pulse DUC – start (3, 2, 3) |
| page | Spec | 16 | G | 2 | 7..0 | General | Used with addresses 16 to 31d, for specific address extension page# |
| En_A0 | C | 17 | G | 3 | 0 | General | Tristate enable A Output[] |
| En_AFS | C | 17 | G | 3 | 1 | General | Tristate enable A port AFS, ACK |
| En_B0 | C | 17 | G | 3 | 2 | General | Tristate enable B Output[] |
| En_BFS | C | 17 | G | 3 | 3 | General | Tristate enable B port BFS, BCK |
| En_C0 | C | 17 | G | 3 | 4 | General | Tristate enable C Output[] |
| En_CFS | C | 17 | G | 3 | 5 | General | Tristate enable C port CFS, CCK |
| En_D0 | C | 17 | G | 3 | 6 | General | Tristate enable D Output[] |
| En_DFS | C | 17 | G | 3 | 7 | General | Tristate enable D port DFS, DCK |
| En_soB | C | 17 | G | 3 | 8 | General, DUC | Tristate enable SyncOut, IFLG |
| ckp_A | C | 17 | G | 3 | 9 | General | Invert ACK |
| ckp_B | C | 17 | G | 3 | 10 | General | Invert BCK |
| ckp_C | C | 17 | G | 3 | 11 | General | Invert CCK |
| ckp_D | C | 17 | G | 3 | 12 | General | Invert DCK |
| overall_gain | P (chan) | T7(*) |  |  |  | DDC | Performs DDC or DUC calculations per channel, and calculates the gain_lsb,gain_msb,fir_shift, cic_shift, cic_rshift, |
| general_timer_sync | U | T68 | xA1 | X12 | 9..7 | General | Sync selection for starting general timer |
| Gen_timer | U | T67 | Xa1 | X11 |  | General | General Timer 16bit counter, can be used for timer sync or sync output |
| gen_timer_test_ | U | T68 | xA1 | X11 | 6 | General | Shortens test counter uses top 16bits, no 256 prescaler |
| Mode AB, CD -receive | P | T7 |  |  |  | General | AB mode, can be differenct from CD mode – receive or transmit |
| syncmode | P | T7(*) |  |  |  | General | Establishes sync mode settings (partial) Used for general setup |
| Cksum_sync _front | U | T68 | xA1 | x12 | 12. 10 | DDC- Receive Input | Sync selection for DUC checksum, DDC LFSR generator |
| rinf_sel_A | C | T1, T66 | xA1 | x10 | 3..0 | DDC- Receive Input | Selects DDC Input port to rinf |
| rinf_sel_B | C | T1, T66 | xA1 | x10 | 7..4 | DDC- Receive Input | Selects DDC Input port to rinf |
| rinf_sel_C | C | T1, T66 | xA1 | x10 | 11..8 | DDC- Receive Input | Selects DDC Input port to rinf |
| rinf_sel_D | C | T1, T66 | xA1 | x10 | 15..12 | DDC- Receive Input | Selects DDC Input port to rinf |
| rin_rate | P (chan) | T1,T7 |  |  |  | DDC- Receive Input | DDC Input rate, 2 double, 1 full, 0 interleaved complex |
| rin_cmplx | P (chan) | T1,T7 |  |  |  | DDC- Receive Input | Complex input – parallel or interleaved |
| rinf_diag | U | T68 | xA1 | x12 | 5..4 | DDC–Receive Input | Selects DDC Input mode, ports, or diagnostics |
| rinf_zpad | U | F4,T68 | xA1 | x12 | 3..0 | DDC–Receive Input | Inserts zpad – 1 zeros in DDC mode if active |
| rinf_zpad_sync | U | F4,T68 | xA1 | x12 | 15..13 | DDC–Receive Input | Sync selection for zpad zero insertion |
| rcv_noise_A | U | T62 | x81 | x12 |  | DDC–Receive Input | Enables noise to be added to specific bit |
| rcv_noise_B | U | T62 | x81 | x12 |  | DDC–Receive Input | Enables noise to be added to specific bit |
| rcv_noise_C | U | T62 | x81 | x12 |  | DDC–Receive Input | Enables noise to be added to specific bit |
| rcv_noise_D | U | T62 | x81 | x12 |  | DDC–Receive Input | Enables noise to be added to specific bit |
| freq | P (chan) | F6,T7 |  |  |  | DDC–Mixer | Calculates fractional frequency based on Fck, and freq |
| Fck | P | F6,T7 |  |  |  | DDC–Mixer | Used with freq for fractional freq calcula |
| mix_rcv_sel | C (chan) | T1,T57 | x80 | x15 | 1..0 | DDC–Mixer | Selects channel from RINF IQ signals |
| mix_rcv_cmplx | C (chan) | T1,T57 | x80 | x15 | 2 | DDC–Mixer | Selects channel from RINF IQ signals |
| mix_icos | C (chan) | F5,T57 | x80 | x15 | 15..14 | DDC–Mixer | Select I for cosine multiply |
| mix_isin | C (chan) | F5,T57 | x80 | X15 | 12..11 | DDC–Mixer | Select I for sine multiply |
| mix_qcos | C (chan) | F5,T57 | x80 | x15 | 9..8 | DDC–Mixer | Select Q for cosine multiply |
| mix_qsin | C (chan) | F5,T57 | x80 | x15 | 6..5 | DDC–Mixer | Select Q for sine multiply |
| mix_inv_icos | C (chan) | F5,T57 | x80 | x15 | 13 | DDC–Mixer | Invert multiply result for subtraction |
| mix_inv_isin | C (chan) | F5,T57 | x80 | x15 | 10 | DDC–Mixer | Invert multiply result for subtraction |
| mix_inv_qcos | C (chan) | F5,T57 | x80 | x15 | 7 | DDC–Mixer | Invert multiply result for subtraction |
| mix_inv_qsin | C (chan) | F5,T57 | x80 | x15 | 4 | DDC–Mixer | Invert multiply result for subtraction, normally 1 |
| phase | U (chan) | F6, T53 | X80 | X11 |  | DDC–Mixer | 0 – 2^16 – 1 is 0 to 2Pi |
| freq_msb | C (chan) | F6, T56 | X80 | X14 |  | DDC–Mixer | Upper 15 bits and sign of 48bit delta phase value |
| freq_mid | C (chan) | F6 | X80 | X13 |  | DDC–Mixer | Middle 16bits of 48bit delta phase value |
| freq_lsb | C (chan) | F6 | X80 | X12 |  | DDC–Mixer | Lower 16bits of 48bit delta phase value |
| freq_sync | C (chan) | F6,T58 | X80 | X16 | 2..0 | DDC–Mixer | Select synchronization for control bus register deltaphase value to update input to phase accumulator |
| phase_sync | C (chan) | F6,T58 | X80 | X16 | 5..3 | DDC–Mixer | Select synchronization for control bus register phase offset value to update dither adder |
| dither_sync | C (chan) | F6, T58 | X80 | X16 | 8..6 | DDC–Mixer | Select synchronization for control bus to restart LFSR sequence for dithered phase |
| nco_sync | C (chan) | F6, T58 | X80 | X16 | 11..9 | DDC–Mixer | Select synchronization for control bus to reset phase accumulator |
| dither_test | C (chan) | T58 | X80 | X16 | 15 | DDC–Mixer | Changes the dither LFSR sequence |
| bypass_mix | P (chan) | T7 |  |  |  | DDC–Mixer | Causes mixer to be programmed for real input to I output, freq 0, phase 0, Gain 2^20-1 / 2^20 |
| cic_shift | C (chan) | T52 | X80 | X19 | 5..0 | DDC–CIC | Used to scale the mixer 21bit output to the 61 bit CIC accumulators, 39bit fixed shift – cic_shift |
| cic_dec | P (chan) | T7 |  |  |  | DDC–CIC | CIC decimation amount |
| ncic | C (chan) | T59 | X80 | X17 | 11..0 | DDC–CIC | CIC decimator modulus (cic_dec – 1) |
| cic_rshift | C (chan) | T52 | X80 | X19 | 9 | DDC–CIC | CIC output scaling for 17 bit output for symmetric PFIR |
| cic_rcv_full | C (chan) | T52 | X80 | X19 | 13 | DDC–CIC | CIC output scaling for 18bit output for non symmetric PFIR |
| cic_rcv | C (chan) | T52 | X80 | X19 | 7..6 | DDC/DUC–CIC | Configures the integrator, comb, decimator, integrator for DDC use |
| cic_2x | C (chan) | T52 | X80 | X19 | 10 | DDC–CIC | Reconfigures the I and Q dual CIC for even and odd CIC usage, enables additional logic between the 2 CICs |
| cic_rcv_cross | C (chan) | T52 | X80 | X19 | 12 | DDC–CIC | Reconfigures the CIC to receive data from the alternate channel mixer, channel B, from channel A-Q; channel D, from channel C-Q |
| flush_sync | C (chan) | F6, T58 | X80 | X16 | 14..12 | DDC–CIC | Sync selection to allow the CIC integrators to zero the feedback for initialization, or inserting zeros |
| cic_sync | U (chan) | F6,T59 | X80 | X17 | 14..12 | DDC–CIC | Sync Selection for cic_dec, to control the synchronization of the cic_decimation counter |
| Cic_bypass | C (chan) | T52 | X80 | X19 | 14 | DDC–CIC | Used for CIC Bypass in Split IQ mode |
| bypass_cic | P (chan) | T7 |  |  |  | DDC–CIC | Only in Split IQ mode, can be used to have a CIC_dec of 1 |
| splitiq | P | T7 |  |  |  | DDC,DUC | Affects CIC and PFIR configuration, indicates that I and Q are processed in separated DDC CIC, PFIR |
| fir_coef | P | T7 |  |  |  | DDC–PFIR | Pseudocommand to accept PFIR tap filename, based on the symmetry, PFIR configuration, loads the taps into the 16 FIR cells |
| Coefficient values | C (chan) |  | Page (0-f) 20,2f 40.4f 60.6f Adr (10-1f) | Page (0-f) 20,2f 40.4f 60.6f Adr (10-1f) | All | DDC–PFIR | PFIR Coefficients by channel, and coefficient number C0(first) – C255 Also sets up PFIR address generators, SwapRAM, BackEndRAM |
| fir_dec | P (chan) | T7 |  |  |  | DDC–PFIR | PFIR decimation amount, provides setup information for the PFIR coefficients separated into the 16 FIR cells, and input for the address generator setup |
| fir_diff | P (chan) | T7 |  |  |  | DDC–PFIR | PFIR different coefficients for I and Q (1) – not used |
| fir_nchan | P (chan) | T7 |  |  |  | DDC–PFIR | PFIR number of data channels in filter (1) – not used |
| fir_sync | U (chan) | T51 | X14 | X15 | 2..0 | DDC–PFIR | Provides synchronization selection for the FIR Controller, normally cic_sync, pfir_sync are the same source |
| coef_sync | U (chan) | T51 | X14 | X15 | 5..3 | DDC–PFIR | Provides coefficient address generator sync selection for the FIR controller, this is normally used at startup, but can be used to dynamically swap between two sets of coefficients, through the address generator offset controls |
| bypass_fir | P (chan) | T7 |  |  |  | DDC–PFIR | causes the system to use a decimate by 1, 16tap asymmetric PFIR, impulse |
|  | C (chan) | None | X10 | All |  | DDC–PFIR | Swap RAM pointer memory – used to re-order the IQ input data from the CIC, currently this is mapped as a bypass |
|  | C (chan) | T18 | X11 | All |  | DDC–PFIR | Backward End cell Control RAM – controls the backward end RAM delay, and blank feedback for end cell for odd symmetric filters |
| coef_ofsc | C | T19 | x12 | x10 | 3..0 | DDC–PFIR | FIR coefficient memory starting address |
| coef_modc | C | T19 | x12 | x10 | 7..4 | DDC–PFIR | FIR coefficient modulus count, start to end |
| coef_repc | C | T19 | X12 | x10 | 11..8 | DDC–PFIR | FIR coefficient address generator control, indicates the number of times the same coefficient is used |
| coef_sym | C | T19 | x12 | x10 | 12 | DDC–PFIR | FIR cell preadd forward and reverse delay lines |
| coef_zerofr | C | T19 | x12 | x10 | 13 | DDC–PFIR | FIR coefficient zero forward read – control is used for interpolation filter, for zero stuffing |
| agen_recr | C | T20 | x12 | x11 | 3..0 | DDC–PFIR | FIR forward delay line recirculation count – number of times the delay line data is reused |
| agen_depd | C | T20 | x12 | x11 | 7..4 | DDC–PFIR | FIR forward delay line depth count – number of delay line elements used |
| agen_modd | C | T20 | x12 | x11 | 11..8 | DDC–PFIR | FIR forward delay line cycle offset – this control is used if multiple signal sets are multiplexed through the filter |
| agen_togen | C | T20 | x12 | x11 | 12 | DDC–PFIR | FIR enable reverse delay – symmetric filter |
| fragen_soff | C | T21 | x12 | x12 | 3..0 | DDC–PFIR | FIR forward delay line read address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| fragen_srecr | C | T21 | x12 | x12 | 7..4 | DDC–PFIR | FIR forward delay line read address – recic counter – starting(sync) recirculation count |
| fragen_soff | C | T21 | x12 | x12 | 11..8 | DDC–PFIR | FIR forward delay line read address – depth counter – starting(sync) depth count |
| fragen_stepn | C | T21 | x12 | x12 | 15.. 12 | DDC–PFIR | FIR forward delay line read address –sample offset starting(sync) step size, indicates the address that the next block of signals will be read from |
| fwagen_soff | C | T22 | x12 | x13 | 3..0 | DDC–PFIR | FIR forward delay line write address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| fwagen_srecr | C | T22 | x12 | x13 | 7..4 | DDC–PFIR | FIR forward delay line write address – recic counter – starting(sync) recirculation count |
| fwagen_soff | C | T22 | x12 | x13 | 11..8 | DDC–PFIR | FIR forward delay line write address – depth counter – starting(sync) depth count |
| fwagen_stepn | C | T22 | x12 | x13 | 15.. 12 | DDC–PFIR | FIR forward delay line write address –sample offset starting(sync) step size, indicates the address that the next block of signals will be written to |
| bragen_soff | C | T23 | x12 | x14 | 3..0 | DDC–PFIR | FIR reverse delay line read address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| bragen_srecr | C | T23 | x12 | x14 | 7..4 | DDC–PFIR | FIR reverse delay line read address – recic counter – starting(sync) recirculation count |
| bragen_soff | C | T23 | x12 | x14 | 11..8 | DDC–PFIR | FIR reversedelay line read address – depth counter – starting(sync) depth count |
| bragen_stepn | C | T23 | x12 | x14 | 15.. 12 | DDC–PFIR | FIR reverse delay line read address –sample offset starting(sync) step size, indicates the address that the next block of signals will be read from |
| bwagen_soff | C | T24 | x12 | x15 | 3..0 | DDC–PFIR | FIR reverse delay line write address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| bwagen_srecr | C | T24 | x12 | x15 | 7..4 | DDC–PFIR | FIR reverse delay line write address – recic counter – starting(sync) recirculation count |
| bwagen_soff | C | T24 | x12 | x15 | 11..8 | DDC–PFIR | FIR reverse delay line write address – depth counter – starting(sync) depth count |
| bwagen_stepn | C | T24 | x12 | x15 | 15.. 12 | DDC–PFIR | FIR reverse delay line write address –sample offset starting(sync) step size, indicates the address that the next block of signals will be written to |
| beragen_soff | C | T25 | x12 | x16 | 3..0 | DDC–PFIR | FIR cell 16 read address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| beragen_srecr | C | T25 | x12 | x16 | 7..4 | DDC–PFIR | FIR cell 16 read address – recic counter – starting(sync) recirculation count |
| beragen_soff | C | T25 | x12 | x16 | 11..8 | DDC–PFIR | FIR cell 16 read address – depth counter – starting(sync) depth count |
| beragen_stepn | C | T25 | x12 | x16 | 15.. 12 | DDC–PFIR | FIR cell 16 read address –sample offset starting(sync) step size, indicates the address that the next block of signals will be read from |
| bewagen_soff | C | T26 | x12 | x17 | 3..0 | DDC–PFIR | FIR cell 16 write address – ‘select sample’ counter – starting(sync) offset, this control is the offset control during synchronization as the starting offset |
| bewagen_srecr | C | T26 | x12 | x17 | 7..4 | DDC–PFIR | FIR cell 16 write address – recic counter – starting(sync) recirculation count |
| bewagen_soff | C | T26 | x12 | x17 | 11..8 | DDC–PFIR | FIR cell 16 write address – depth counter – starting(sync) depth count |
| bewagen_stepn | C | T26 | x12 | x17 | 15.. 12 | DDC–PFIR | FIR cell 16 line write address –sample offset starting(sync) step size, indicates the address that the next block of signals will be written to |
| fwa_strobe |  | T27 | x12 | x18 |  | DDC–PFIR | 16 element times to write to next forward FIR cell |
| bwa_strobe |  | T28 | x12 | x19 |  | DDC–PFIR | 16 element times to write to next reverse FIR cell |
| Beagen_rbypass |  | T29 | x12 | x1a |  | DDC–PFIR | 16 element times to indicate Backend RAM bypass delay |
| Swap_wa_cnt |  | T46 | x14 | x10 | 3..0 | DDC–PFIR | FIR Swap RAM Write address count |
| Swap_ra_cnt |  | T46 | x14 | x10 | 7..4 | DDC–PFIR | FIR Swap RAM Read address count |
| Swap_xmt |  | T46 | x14 | x10 | 8 | DDC–PFIR | Swap RAM mode Transmit or receive |
| Swap_cmplx |  | T46 | x14 | x10 | 9 | DDC–PFIR | Swap RAM data is interleaved complex or Real |
| Fir_fb |  | T46 | x14 | x10 | 10 | DDC–PFIR | FIR forward broadcast |
| Swap_fb |  | T46 | x14 | x10 | 11 | DDC–PFIR | SwapRAM forward broadcast |
| Swap_rcv_tdly |  | T46 | x14 | x10 | 12 | DDC–PFIR | FIR swap RAM delay control for cic bypass |
| Acc_dly |  | T47 | x14 | x11 | 3..0 | DDC–PFIR | PFIR_Accumulator delay |
| Acc_cnt |  | T47 | x14 | x11 | 7..4 | DDC–PFIR | PFIR Accumulator, number of accumulations per cycle |
| Acc_dly0 |  | T47 | x14 | x11 | 8 | DDC–PFIR | FIR accumulator cycle delay |
| Acc_bypass |  | T47 | x14 | x11 | 9 | DDC–PFIR | Accumulator bypass – no partial accumulation |
| Acc_enram |  | T47 | x14 | x11 | 10 | DDC–PFIR | Enable partial accumulation RAM, in feedback path |
| fir_shift | C (chan) | T48 | X14 | X12 | 2..0 | DDC–PFIR | PFIR accumulator output (42bits) is shifted to an 18bit output, Fir shift has a variable 8 bit shift, along with a fixed shift. |
| Fir_xenq | C (chan) | T48 | X14 | X12 | 3 | DDC–PFIR | FIR transmit enable Q |
| Fir_xeni | C (chan) | T48 | X14 | X12 | 4 | DDC–PFIR | FIR transmit enable I |
| Fir_half | C (chan) | T48 | X14 | X12 | 5 | DDC–PFIR | Set to 0 |
| Fir_rnd20 | C (chan) | T48 | X14 | X12 | 6 | DDC–PFIR | 1 for 20bit receive, 0 for 18bit transmit |
| Fir_cmplx | C (chan) | T48 | X14 | X12 | 7 | DDC–PFIR | FIR outputs complex interleaved data |
| Fir_sync_cnt | C (chan) | T49 | X14 | X13 |  | DDC-PFIR | Periodic FIR update counter |
| Fir_clk_cnt | C (chan) | T50 | X14 | X14 | 7..0 | DDC | Number of FIR counts per CIC output - 1 |
| Fir_clk_cnt_ena | C (chan) | T50 | X14 | X14 | 8 | DDC | 1 for normal, 0 for always |
| Fir_test | C (chan) | T50 | X14 | X14 | 9 | DDC | 0 for normal |
| Fir_en_sync_cnt | C (chan) | T50 | X14 | X14 | 10 | DDC | Enable fir sync counter |
| pwr_mtr_sync | U (chan) | T51 | X14 | X15 | 11..9 | DDC – Power Meter | Sync selection for the power meter integration counter start |
| pwr_meter_on | P | T7(*) |  |  |  | DDC – Power Meter | Power meter configuration |
| pwr_integ | P | T7(*) |  |  |  | DDC – Power Meter | Power meter integration period setup, the counter counts I,Q samples, after integrating the specificied number of samples the accumulated value is registered |
| Pwr_mtr_integ | U (chan) | T43 | X13 | X1d |  | DDC – Power Meter | Power meter integration down counter in IQ counts |
| pwr_test | U (chan) | T7(*) | T44 | X13 | X1e | DDC – Power Meter | Power Meter test, 0 for receive |
| gain | P | T7 |  |  |  | DDC– Gain, AGC | Variable used to set the gain_lsb, gain_msb; value is relative to 1.0 for gain of 4096 |
| gain_sync | U (chan) | T51 | X14 | X15 | 8..6 | DDC– Gain, AGC | Used to synchronize the gain register to manual gain update. AGC uses the gain_sync to re-zero the adaptive gain |
| gain_lsb | C (chan) | T33 | X13 | X13 |  | DDC– Gain, AGC | Lower 16bits of 19bit gain word. This is the manual gain. |
| gain_msb | C (chan) | T34 | X13 | X14 | 2..0 | DDC– Gain, AGC | Upper 3 bits of 19bit gain word. This is the manual gain. |
| agc_freeze | U (chan) | T34 | X13 | X14 | 3 | DDC– Gain, AGC | See AGC application note |
| gain_rnd | C (chan) | T34 | X13 | X14 | 8..4 | DDC– Gain, AGC | Sets rounding for DDC output mode, 20, 16, 12, 8 bits |
| Gain_half | C (chan) | T34 | X13 | X14 | 9 | DDC– Gain, AGC | 0 for receive |
| agc_hold | U (chan) | T34 | X13 | X14 | 10 | DDC– Gain, AGC | ‘1’ to hold adaption gain |
| agc_zmag | U (chan) | T34 App Note | X13 | X14 | 15..12 | DDC– Gain, AGC | See AGC application note |
| agc_min | U (chan) | T35, AppNote | X13 | X15 |  | DDC– Gain, AGC | See AGC application note |
| agc_max | U (chan) | T36, AppNote | X13 | X16 |  | DDC– Gain, AGC | See AGC application note |
| agc_zero_cnt | U (chan) | T37 AppNote | X13 | X17 | 3..0 | DDC– Gain, AGC | See AGC application note |
| agc_sat_cnt | U (chan) | T37 AppNote | X13 | X17 | 7..4 | DDC– Gain, AGC | See AGC application note |
| agc_thresh | U (chan) | T37 AppNote | X13 | X17 | 15..8 | DDC– Gain, AGC | See AGC application note |
| agc_Dblw | U (chan) | T38 AppNote | X13 | X18 | 3..0 | DDC– Gain, AGC | See AGC application note |
| agc_Dadv | U (chan) | T38 AppNote | X13 | X18 | 7..4 | DDC– Gain, AGC | See AGC application note |
| agc_Dzro | U (chan) | T38 AppNote | X13 | X18 | 11..8 | DDC– Gain, AGC | See AGC application note |
| agc_Dsat | U (chan) | T38 AppNote | X13 | X18 | 15..12 | DDC– Gain, AGC | See AGC application note |
| pins | P | T7 |  |  |  | DDC– Receive Output | 4,8,16 – active pins on output port, determines the output signal packet, with the number of bits |
| bits | P | T7 |  |  |  | DDC– Receive Output | 4,8,12,16,20 – bits in each word output data – used to control the AGC rounding at the gain output, and the number of bits which are transmitted, n pins at a time |
| sck_sync | U (chan) | T51 | X14 | X15 | 14..12 | DDC– Receive Output | Sync selection for sck_divider counter, should match the cic_sync and fir_sync selection if used |
| routf_tdm | P | T7 |  |  |  | DDC– Receive Output | Indicated DDC output is TDM mode for port D, if zero either interleaved IQ or real data can be output |
| Routf_pins | C (chan) | T44 | X13 | X1e | 2..0 | DDC– Receive Output | Receive output pin select, 4, 8, or 16 |
| Routf_bits | C (chan) | T44 | X13 | X1e | 7..3 | DDC– Receive Output | Receive output bits select, 4,8,12,16, or 20 |
| routf_iqmux | C (chan) | T44 | X13 | X1e | 8 | DDC– Receive Output | ‘1’ for interleaved IQ output , ‘0’ for real |
| Routf_pwrdown | C (chan) | T44 | X13 | X1e | 9 | DDC– Receive Output | ‘0’ in receive mode |
| Routf_ctdm | C (chan) | T44 | X13 | X1e | 11 | DDC– Receive Output | ‘1’ for TDM IQ output mode |
| agc_gain_out | U (chan) | T34 | X13 | X14 | 11 | DDC– Receive Output | ‘1’ to change IQ output to include current gain value |
| sck_div | U (chan) | T31 | 13 | x11 | 11.8 | DDC– Receive Output | Controls the divided channel clock, Frame Strobe and DDC output data width Divides the ACK, BCK, CCK, DCK by (sck_div + 1), used in both DDC and DUC modes |
| Toutf_xmtCD | C | T70 | Xa1 | X14 | 12 | Misc | ‘0’ for DDC mode |
| Toutf_xmtAB | C | T70 | Xa1 | X14 | 14 | Misc | ‘0’ for DDC mode |

Notes: (chan) – indicates that the variable is repeated on 4 different pages if the variable is individually controlled.  Variables that are shared between AB or CD have separate bit controls, and two addresses.  See Table 8 in the data sheet for the mapping of channels to tables.  The table indicates the common addresses and Channel A.

Legend

P – Pseudocommand,  SU – Startup Controlled, manual sequence

C – computed,	Spec – Page register is written each time that a different page of registers is accessed.

U – user provided