# Serial Flashing on OMAP3530 and AM35X EVM

## Host Side Software Utility

- download host PC utility  http://omap-u-boot-utils.googlecode.com/files/omap-u-boot-utils-pack-r0.2.tgz

- unzip the omap-u-boot-utils.tag-v0.2.0.tgz included inside the downloaded tarball to, for example , C:\ .

- You should now have directory C:\nmenon-omap-u-boot-utils-1a51d6b5c5947ad94c58c266c4e735fd776d30d8

- Rename the above directory to, say, C:\omap-u-boot-utils.  This step is only optional.  The only purpose is to rename to a shorter directory name.

- Follow instruction in the “Windows” section in C:\omap-u-boot-utils\README to install MinGW gnu compiler and GnuWin32 applications.  This may take a while, but should be significantly less time than installing WinCE.

- Follow instructions in the “Windows” section in C:\omap-u-boot-utils\README to set up Windows Path environment variable.  You may need to reboot the PC afterwards to allow the new Path to take effect.

- After PC reboot, start a cmd window

- cd C:\omap-u-boot-utils

- make

- This should generate “pserial.exe” and other executables.  We only need pserial.exe

## Target Side Software

- Make sure your target board BSP has the XLDRUART update.  If not unzip the corresponding zip file to update the BSP.  Check directory for corresponding BSP installed

  - C:\WINCE600\PLATFORM\AM35x_BSP\SRC\BOOTLOADER\XLDR\UART

  - C:\WINCE600\PLATFORM\TI_EVM_3530\SRC\BOOT\XLDR\UART

- Build the BSP.   Depending on the BSP installed, this should generate either one of the two sets of the following files in the corresponding build release directory  ( *-nand.raw contains 4 copies of XLDRNAND.nb0 and 1 copy of EBOOTNAND.nb0 )

  - XLDRUART.nb0 and AM35x-nand.raw for AM35X EVM

  - XLDRUART.nb0 and TIEVM3530-nand.raw for OMAP3530 EVM

## Download Image File to AM35X EVM

- On EVM, make sure S7-1 and S7-4 are ON, all other S7 pins are OFF.

- Connect a serial cable to PC’s COM1.

- Copy XLDRUART.nb0 and AM35X-nand.raw to C:\omap-u-boot-utils

- On PC, start a cmd window

- cd C:\omap-u-boot-utils

- pserial.exe –p COM1 –f XLDRUART.nb0

- Power up EVM.  Should now see download progress.

- After this download is done (when “File download completed” is shown), keep the EVM power up.

- Bring up HyperTerminal:  Start > All Programs > Accessories > Communications > HyperTerminal

- Configure COM1 to 115200:8:None:1:None

- You should now see “?” every 10 seconds on HyperTerminal

- On HyperTerminal, choose Transfer > Send File …

- Choose Xmodem from drop down menu on Dialog box.

- Browse to C:\omap-u-boot-utils\AM35X-nand.raw

- Click “Send” button

- Should start seeing send progress within 10 sec.  This download takes about 3 min.

- After download is completed, power off EVM.

- Keep pin S7-4 ON and put all other S7 pins to OFF.

- Power up EVM.  Should now see EBOOT boots up.

## Download Image File to OMAP3530 EVM

- On EVM, make sure S4-2 and S4-4 are ON, all other S7 pins are OFF.

- Connect a serial cable to PC’s COM1.

- Copy XLDRUART.nb0 and TIEVM3530-nand.raw to C:\omap-u-boot-utils

- On PC, start a cmd window

- cd C:\omap-u-boot-utils

- pserial.exe –p COM1 –f XLDRUART.nb0

- Power up EVM.  Should now see download progress.

- After this download is done (when “File download completed” is shown), keep the EVM power up.

- Bring up HyperTerminal:  Start > All Programs > Accessories > Communications > HyperTerminal

- Configure COM1 to 115200:8:None:1:None

- You should now see “?” every 10 seconds on HyperTerminal

- On HyperTerminal, choose Transfer > Send File …

- Choose Xmodem from drop down menu on Dialog box.

- Browse to C:\omap-u-boot-utils\TIEVM3530-nand.raw

- Click “Send” button

- Should start seeing send progress within 10 sec.  This download takes about 3 min.

- After download is completed, power off EVM.

- Power up EVM.  Should now see EBOOT boots up.