Boot Image Handling Utilities for C6474

The following provides a brief description of boot image handling utilities for C6474 in aim for customers to boot the device quickly. It is suggested that customer read the C6474 bootloader user’s guide to understand how booting works on TCI648x device.

# Introduction to Utilities

| Utilities | Description |
| --- | --- |
| b2ccs.exe | Convert a boot table into ccs dat file format plus a header for I2C EEPROM programming using eeprom.out |
| b2i2c.exe | Convert a boot table into multiple blocks, each block has 0x80 bytes with a length and checksum in the beginning. |
| bconvert64x.exe | Convert a hex6x boot table file into the format required by the TCI648x chip boot loader which expects all 32 bit values to be in big endian. Typically this program handles conversion of data values that are in sections that are not multiples of 4 bytes. |
| bootpacket.exe | Convert a boot table to an Ethernet packet format for boot. Destination and source MAC address can be specified. For example: bootpacket simple.btbl.be simple.eth 00-01-02-03-04-05 0A-0B-0C-0D-0E-0F |
| hex6x.exe | Code gen tool, convert a .out file to a boot table binary format. |
| mergebtbl.exe | Merge three boot tables for three cores into just one. |
| myparser.exe | Merge I2C boot parameters and boot tables |
| eeprom_0x50.out | I2C EEPROM programming utility. Must use it with boot mode 0 as PLL1 is supposed to be in bypass mode(check register 0x29a0100), target slave address is 0x50. |
| eeprom_0x51.out | I2C EEPROM programming utility. Must use it with boot mode 0 as PLL1 is supposed to be in bypass mode(check register 0x29a0100), target slave address is 0x51. |

# Introduction to Process

Customer need to prepare all the input files and in some cases modify the .bat file to get the final output file.  *.out file is created from customer’s application, note global address must be used in the command file for non- core 0. For *.rmd file, refer to SPRU186P, Section 11, hex conversion utility description for more details. It’s suggested customer try the example boot first to just verify the environment before the final image is used.

## Example 1: I2C Master boot using one .out file

One simple.out could be the image for either GEM0 only or all three GEMS. The following figure shows the interface:

Input:

simple.out

simple.rmd

i2cconfig.txt

Output:

I2cromfara.ccs

Utilities

Here is what is in I2Cboot\I2Cboot.bat:

hex6x simple.rmd

bconvert64x -le simple.btbl simple.btbl.be

b2i2c simple.btbl.be simple.btbl.i2c

b2ccs simple.btbl.i2c simple.i2c.ccs

myparser simple.i2c.ccs i2cconfig.txt i2cromfara.ccs

## Example 2: I2C Master boot using three .out files

Each .out file is for particular GEM.

Input:

Core0.out

Core0.rmd

Core1.out

Core1.rmd

Core2.out

Core2.rmd

i2cconfig.txt

Output:

I2cromfara.ccs

Utilities

Here is what is in I2Cboot_3core\I2Cboot_3core.bat

hex6x core0.rmd

hex6x core1.rmd

hex6x core2.rmd

mergebtbl core0.btbl core1.btbl core2.btbl simple.btbl

bconvert64x -le simple.btbl simple.btbl.be

b2i2c simple.btbl.be simple.btbl.i2c

b2ccs simple.btbl.i2c simple.i2c.ccs

myparser simple.i2c.ccs i2cconfig.txt i2cromfara.ccs

## Example 3: EMAC boot using one .out file

One simple.out could be the image for either GEM0 only or all three GEMs.

Input:

simple.out

simple.rmd

Output:

Simple.eth

Utilities

Here is what is in the EMACboot\EMACboot.bat

hex6x simple.rmd

bconvert64x -le simple.btbl simple.btbl.be

bootpacket simple.btbl.be simple.eth

## Example 4: EMAC boot using three .out file

One simple.out could be the image for either GEM0 only or all three GEMs.

Input:

Core0.out

Core0.rmd

Core1.out

Core1.rmd

Core2.out

Core2.rmd

Output:

Simple.eth

Utilities

Here is what is in the EMACboot\EMACboot_3core.bat

hex6x core0.rmd

hex6x core1.rmd

hex6x core2.rmd

mergebtbl core0.btbl core1.btbl core2.btbl simple.btbl

bconvert64x -le simple.btbl simple.btbl.be

bootpacket simple.btbl.be simple.eth

## Example 5: EMAC PC boot for C6474 EVM

PC

C6474

HUB/Switch

The idea is to boot in I2C mode first which will configure PHY on the EVM board, and then jump to EMAC boot mode. The I2C EEPROM also contains a customized SerDes configuration for EMAC. So need to program I2C EEPROM with the .ccs file under I2Cboot_otherboot directory. Then also need to create a application image that need to be booted which is shown in example 3 and 4. Finally the PC utility will be used to send packet over.  Note currently the pcsendpkt.exe utility doesn’t wait for the bootp packet (Ethernet ready announcement) so if boot table packet is sent before DSP is ready, the boot will fail.

The following are detailed steps:

Note: There is a known problem with Faraday EVM that auto-negotiation does not work, so forced mode to 100Mbps or 1Gbps should be used.

1. Remove .gel file in your ccs setup. Some .gel file will configure PLL1 and reset DSP automatically which may not be good for debugging.

2. Power off the board and put the DSP in no-boot mode

3. Power up board

4. Program I2C:

- Open CCS and one core (say core 0 on the first device)

- Debug->Connect

- set PLL to bypass mode by setting address 0x29a0100 to 0x40

- File->Load Program -> Boottest/Utilities/eeprom_0x50.out

- File->Data->Load -> I2cboot_otherboot\I2cboot_emac_evm_autonegoDisabled_DEVNUM_equals_to_1_PG1.1_1Gbps.ccs if 1Gbps network is used and EVM revision is Rev F or G.  Several examples provided in I2Cboot_otherboot directory, if 10/100Mbps network is used and Rev D and below EVM is used, select I2cboot_emac_evm_autonegoDisabled_DEVNUM_equals_to_0_PG1.0_100Mbps.ccs, DEVNUM can be read from DEVSTAT register(0x02880804), PG number can be read from VARIAN field of DEVID register(0x02880814)

- View -> Watch window -> click on Watch 1 tab, enter "status", and change the radix to "hex"

- Debug -> Run.

- After say 10 sec, Halt. "status" should be 0x55AA (means I2C programming succeeded).

5. Close CCS, Power off the board, change to bootmode 1

6. Power up the board

7. From a DOS prompt, run Boottest/EMACboot_pc/arpit.bat (the IP address needs to be on the local subnet. The MAC address already matches what's in the .ccs file.

8. Send the application image to the DSP: From the same DOS prompt, run Boottest/EMACboot_pc/pcsendpkt simple.eth. It should say "Sent 1 packet". If it says "Send failed" then you'll need to change the IP address and make sure it's there using arp -a.

9. Use CCS, Debug->Connect. Register A1 = 0X11223344.

The PC should be at the following line:

00810024 0001A120            BNOP.S1       0x810024 (PC+4 = 0x00810024),5

If LedBlink.eth is used,a LED will blink so you don’t need CCS.

## Example 6: SRIO boot

Step 1: build DSP code

build dspcode under code composer studio, the target is DspCode.out, refer to DspCode_basic directory. The example is using little endian.

Step 2: prepare the DSP image file for boot

option a) core 0 only, refer to DspImage directory, execute the run.bat under windows, the target is DspCode.h which can be included as a header file.

option b) three cores, refer to DspImage_3core directory, execute run.bat under windows, the target is DspCode.h which can be included as header file.

The procedure can be illustrated as below:

Core0.out

Core0.rmd

Core1.out

Core1.rmd

Core2.out

Core2.rmd

Hex6x

Hex6x

Hex6x

Core0.btbl

Core2.btbl

Core1.btbl

M

E

R

G

E

B

T

B

L

DspCode.btbl

Bttbl2Hfile

DspCode.h

DspCode.bin (binary)

option c) if Motorola S Record file format is desired, refer to SRecord or SRecord_3core,  execute run.bat, the target is DspCode.srec.

Step 3: Boot DSP

option a) from another C6474: refer to Boot_From_Another C6474 directory

option b) from any Microprocessor: refer to Boot_From_Microprocessor directory

# FILE formats

I2cromfara.ccs has the following format:

1st line: CCS data format

2nd line: length in bytes, not include itself and 3rd line

3rd line: start address of I2C EEPROM to program

4th line and the rest: I2C boot parameter & boot tables

Simple.eth has the following format:

1st line: CCS data format

2nd line: 0x0000

3rd line: length of first packet in bytes, length counts not include itself

.......

first packet

.......

0xEA00

0x0000

length of the second packet in bytes, length counts not include itself

.......

second packet

.......

0xEA00

0x0000

length of the other packet in bytes, length counts not include itself

.......

other packets

.......

0xEA00

0X0000

0X0000: end of the file

# Switch to Big Endian

The action will be as follows:

- create the .out in big endian(build option –me)

- change the -order to big endian in *.rmd file (-order M)

- In I2C boot batch file, bconvert64x has –le for little endian, -be for big endian.

- In some example(SRIO), note SRIO transfer is always in big endina.

# How to use I2C EEPROM Programming Utility

For boot debugging purpose, it is recommended to remove the .gel file in setup as some .gel file will automatically reset DSP or setup PLL1 when you connect to CCS.  If PLL1 is programmed, changed the content of register 0x29a0100 to 0x40 to make PLL1 become bypassed.

For example, if boot mode 1 is used, boot the device in no boot option, then connect CCS, load eeprom_0x50.out, then do a Data Load  I2Cromfara.ccs, then open a watch window for variable status, run the program until status = 0x55aa.

# Source Code for the Utilities

The source code for all the utilities are in the Program utility_srcDSP. The I2C programming utility source code is in Program utility_srcI2C_programmer. The source code for the pcsendpkt is located in Program utility_srcPC folder.