/*	 @test name 	arm_mcbsp_loop_txii_rxii_poll	*/

/*************************************************************************/

/*

**

** \note The values which are not selected in select mask can be

**       initialized to any legal value for their data type as they will

**       not be used for McBSP configuration.

**

** \warning Executing CSL_MCBSP_CMD_REG_RESET will reset entire McBSP. Hence

**          each test case initialization should be self sufficient and

**          should not depend on any previously initialized value.

**

** \Register Configurations:

** - Test Case 1:

**   -# No Of Phase - 1,

**   -# Word Length - 32,

**   -# Frame Length - 1,

**   -# Data Delay  - 2,

**   -# Interrupt on Transmitter and Receiver ready,

**   -# internal Transmit clock and Frame Sync,

**   -# Digital Loop Back Mode ON,

**   -# Internal Frame Sync Generation from Sample Rate Generator

**   -# Frame Sync Pulse Width  - 1,

**   -# Frame Sync Period - DXR TO XSR COPY

**   -# Clock Divide Value - 0xFF,

*************************************************************************/

/** Header files corresponding to MCBSP module*/

#include <csl_mcbsp.h>

/*   mcbspCommon.h contains common variable declarations and function calls. */

#include <mcbsp_common.h>

#include <csl_deviceconfig.h>

#include <cslr_deviceconfig_001.h>

#define NUM_ELEMENTS	32

/*************************************************************************/

/** \brief   An array of values to be transmitted.

*************************************************************************

** This array consists of values which will be transmitted out of McBSP.

** It will be initialized based on the transmit word length. These values

** can be compared with values received by testbench or same McBSP(in

** case of loop back).

*************************************************************************/

#pragma DATA_SECTION (xmitBuff, ".DDREMIF_0_BUF");

Uint32 xmitBuff[NUM_ELEMENTS];

/*************************************************************************/

/** \brief   An array of received values.

*************************************************************************

** This array consists of values which are received by McBSP.

** These values can be compared with values transmitted by testbench or

** same McBSP(in case of loop back).

*************************************************************************/

#pragma DATA_SECTION (rcvBuff, ".DDREMIF_0_BUF");

Uint32 rcvBuff [NUM_ELEMENTS];

#pragma DATA_SECTION (mcbspDebug, ".DDREMIF_1_BUF");

Uint32  mcbspDebug [10];

//  Used to initialize the array of data to be transmitted

Uint16 tx_word_length;

// This value is used for initialization of array of values to be transmitted

Uint16 val4bit= 0x1;

int Sample_rate_ClkDivide		= 0x2; //Due to QTclk/1

int Sample_rate_framePeriod     = 0x40;

int Sample_rate_pulseWidth     = 0x1;

mcbsp_doTest()

{

Uint32 i=0, j=0, k;

CSL_McbspRegsOvly mcbspRegs;

// CSL system Initialization

CSL_sysInit();

//Initialize the MCBSP CSL

status = CSL_mcbspInit();

if(status != CSL_SOK)

failFlag++;

//setup pin muxing in order to view signals on pins

VAL_pinMux(pinMuxReg1, CSL_PINMUX1_McBSP_ENABLE);

// Open Call for Mcbsp0

hMcbsp[i] = CSL_mcbspOpen(&myMcbspObj[i],CSL_MCBSP_PERI[i],CSL_EXCLUSIVE,NULL,&status);

mcbspRegs = hMcbsp[0]->regs;

if(status != CSL_SOK)

failFlag++;

// Reset the MCBSP Channel

CSL_mcbspHwControl (hMcbsp[i], CSL_MCBSP_CMD_REG_RESET, 0);

/*************************************************************************/

/* \brief  Configure the MCBSP with following values.

*************************************************************************

** \note McBSP receiver, transmitter frame sync logic and sample rate

** generator are not enabled in this function. This is done because

** once these are enabled, McBSP will start its operation, which might

** not be required till DMA and IRQ setups, if any are done.

*************************************************************************/

myglobal.ioEnableMode		= CSL_MCBSP_IOMODE_TXDIS_RXDIS;

myglobal.dlbMode			= CSL_MCBSP_DLBMODE_ON;

myglobal.clkStopMode		= CSL_MCBSP_CLKSTP_DISABLE;

myrxdata.phaseNum 			= CSL_MCBSP_PHASE_SINGLE;

myrxdata.wordLength1 	    = CSL_MCBSP_WORDLEN_32;

myrxdata.frmLength1         = 1;

myrxdata.wordLength2		= CSL_MCBSP_WORDLEN_32;

myrxdata.frmLength2  	    = 0;

myrxdata.frmSyncIgn	 		= CSL_MCBSP_FRMSYNC_DETECT;

myrxdata.compand 			= CSL_MCBSP_COMPAND_OFF_MSB_FIRST;

myrxdata.dataDelay 			= CSL_MCBSP_DATADELAY_2_BITS;

myrxdata.rjust_dxenable 	= CSL_MCBSP_RJUSTDXENA_RJUST_RZF;

myrxdata.IntEvent 			= CSL_MCBSP_INTMODE_ON_READY;

mytxdata.wordReverse 		= CSL_MCBSP_32BIT_REVERS_DISABLE;

mytxdata.phaseNum 			= CSL_MCBSP_PHASE_SINGLE;

mytxdata.wordLength1 	    = CSL_MCBSP_WORDLEN_32;

mytxdata.frmLength1         = 1;

mytxdata.wordLength2		= CSL_MCBSP_WORDLEN_32;

mytxdata.frmLength2  	    = 0;

mytxdata.frmSyncIgn	 		= CSL_MCBSP_FRMSYNC_DETECT;

mytxdata.compand 			= CSL_MCBSP_COMPAND_OFF_MSB_FIRST;

mytxdata.dataDelay 			= CSL_MCBSP_DATADELAY_2_BITS;

mytxdata.rjust_dxenable 	= CSL_MCBSP_RJUSTDXENA_RJUST_RZF;

mytxdata.IntEvent 			= CSL_MCBSP_INTMODE_ON_READY;

mytxdata.wordReverse 		= CSL_MCBSP_32BIT_REVERS_DISABLE;

myclkset.frmSyncTxMode 		= CSL_MCBSP_FSCLKMODE_INTERNAL;

myclkset.frmSyncRxMode		= CSL_MCBSP_FSCLKMODE_EXTERNAL;

myclkset.clkRxMode			= CSL_MCBSP_TXRXCLKMODE_EXTERNAL;

myclkset.clkTxMode			= CSL_MCBSP_TXRXCLKMODE_INTERNAL;

myclkset.frmSyncTxPolarity	= CSL_MCBSP_FSPOL_ACTIVE_HIGH;

myclkset.frmSyncRxPolarity	= CSL_MCBSP_FSPOL_ACTIVE_HIGH;

myclkset.clkTxPolarity		= CSL_MCBSP_CLKPOL_TX_RISING_EDGE;

myclkset.clkRxPolarity		= CSL_MCBSP_CLKPOL_RX_FALLING_EDGE;

myclkset.srgFrmPulseWidth	= Sample_rate_pulseWidth;

myclkset.srgFrmPeriod		= Sample_rate_framePeriod;

myclkset.srgClkDivide		= Sample_rate_ClkDivide;

myclkset.srgInputClkMode	= CSL_MCBSP_SRGCLK_CLKCPU;

myclkset.srgClkPolarity		= CSL_MCBSP_CLKPOL_SRG_RISING_EDGE;

myclkset.srgTxFrmSyncMode	= CSL_MCBSP_TXFSMODE_DXRCOPY;

myclkset.srgClkSync			= CSL_MCBSP_CLKGSYNCMODE_OFF;

mymulCh.rxPartition			=CSL_MCBSP_PARTMODE_2PARTITION;  //RMCME

mymulCh.txPartition			=CSL_MCBSP_PARTMODE_2PARTITION;	 //XMCME

mymulCh.rxMulChSel			= 0x0;		//RMCM

mymulCh.txMulChSel			= 0x0;		//XMCM

mymulCh.rxPartABlk			=CSL_MCBSP_PABLK_0;

mymulCh.rxPartBBlk			=CSL_MCBSP_PBBLK_1;

mymulCh.txPartABlk			=CSL_MCBSP_PABLK_0;

mymulCh.txPartBBlk			=CSL_MCBSP_PBBLK_1;

mymulCh.rx512channel		=0x0;		//REMODE

mymulCh.tx512channel		=0x0;		//XEMODE

mySetup.emumode					=CSL_MCBSP_EMU_STOP;

mySetup.dxState					=CSL_MCBSP_DXSTATE_DEFAULT;

mySetup.rxsynccount				=CSL_MCBSP_RSYNCCNT_DEFAULT;

mySetup.txsynccount				=CSL_MCBSP_XSYNCCNT_DEFAULT;

mySetup.global = &myglobal;

mySetup.rxdataset = &myrxdata;

mySetup.txdataset = &mytxdata;

mySetup.clkset = &myclkset;

mySetup.mulCh = &mymulCh;

//Configure the Mcbsp registers

CSL_mcbspHwSetup(hMcbsp[0], &mySetup);

/* McBSP data transmit wordlength. This is used to initialize the array of data

to be transmitted */

tx_word_length = (Uint16)mytxdata.wordLength1;

/* In case of companding, even though transmit word length is 8 bits,

The data should be written as 16 bits */

if((mytxdata.compand == 0x2) ||

(mytxdata.compand == 0x3))

tx_word_length = 2;

/* Initialization of transmit values and array of received values */

for(k=0;k<NUM_ELEMENTS;k++)

{

/* Initialize the array of received values to all 0s */

rcvBuff[k] = 0;

/* Initialize the array of values to be transmitted based on the word length */

xmitBuff[k] = val4bit;

for(j=1;j<=(tx_word_length + 2);j++)

{

xmitBuff[k] |= (Uint32)val4bit << (j*4);

}

if(mytxdata.compand == 0x2)

xmitBuff[k] &=0xfffc;

if(mytxdata.compand == 0x3)

xmitBuff[k] &=0xfff8;

val4bit++;

if(val4bit > 0xf)

val4bit = 0;

}

/* Start Sample Rate Generator Clock Generation */

grstControl (enable);

/* Start Sample Rate Generator Frame Generation */

frstControl (enable);

/* Enable Receiver */

rrstControl  (enable);

/* Enable Transmitter */

xrstControl (enable);

j = 0;  //loop counter

for(j=0;j<NUM_ELEMENTS ;j++)

{

/* Poll for XRDY flag to acknowledge transmitter  is ready */

while (!CSL_FEXT(mcbspRegs->SPCR, MCBSP_SPCR_XRDY));

CSL_mcbspWrite (hMcbsp[i], CSL_MCBSP_WORDLEN_32, &xmitBuff[j]);

/* Poll for RRDY flag to acknowledge receiver is ready */

while (!CSL_FEXT(mcbspRegs->SPCR, MCBSP_SPCR_RRDY));

CSL_mcbspRead (hMcbsp[i], CSL_MCBSP_WORDLEN_32, &rcvBuff[j]);

}

// Compare the transmitted data and received data to detemine whether test case has passed.

for(j=0;j<NUM_ELEMENTS;j++)

{

if(xmitBuff[j]!=rcvBuff[j])

{

mcbspDebug[j]   = (xmitBuff[j] );

mcbspDebug[j+1] = (rcvBuff[j] );

mcbspDebug[j+2] = j;

failFlag++;

break;

}

}

// Reset the MCBSP Channel

CSL_mcbspHwControl (hMcbsp[0], CSL_MCBSP_CMD_REG_RESET, 0);

//disable MCBSP pin muxing

VAL_pinMux(pinMuxReg1, CSL_PINMUX1_McBSP_DISABLE);

CSL_mcbspClose (hMcbsp[0]); //close mcbsp

// Determine if any failFlag have been encountered and returns test case result to Main

if(!failFlag)

{

return 0;

}

else

{

return 1;

}

}