Details:

Below, the section numbers (ie 1.1.1) refer to the relevant section of the spec, highlights in green indicated that the layout appears to conform to the layout spec, highlights in yellow indicated the layout likely conforms to the layout spec, but the customer should check, highlights in red are spec violations and should be corrected.

1.1.1:  DDR2 Device Combinations – OK

DDR device configuration appears valid.  There are 2 x16-bit DDR2 memories to make up 32-bit DDR2 interface.

1.1.2: Compatible JEDEC DDR2 Devices - Assumed OK, customer should verify.

1.1.3: PCB Stackup – need confirmation

1 – Top DDR2 Routing plus other routing

2 – Appears to be full ground plane

3 – Appears to 1.8V DDR2 power plane plus other routing, please confirm

4 – Appears to be internal signal laying in the ground plane, please confirm

5 – Appears to be split power and ground plane, please confirm

6 – Bottom DDR2 routing plus other routing

Customer should verify BGA pad sizes are correct for DSP device as well as DDR devices.

1.1.4: Placement - OK

1.1.5: DDR Keepout Region – OK

Didn’t find any Non-DDR signals on the DDR signal layer within this region.

1.1.6: Bulk Bypass Capacitors – violated

C65, C225 and C57 have value of 22uF in the schematics, while showed 1uF in the layout.

Customer needs to make sure if there are other mismatches.

1.1.7: High Speed Bypass Capacitors - violated

Found 10 for the processer, expect 11.

Found 8 for each DDR device, OK

1.1.8:  Return Current Bypass Capacitors – violated

Didn’t find any. Make sure when return current reference plane changes, bypass capacitors need to be applied.

1.1.9:  DDR2 signal termination   – OK

1.1.10:  VREFSSTL_DDR Routing   – violated

VREF is 15 mils wide, should be 20 mils wide.

1.1.11:  CK and ADDR_CTRL Topology and Routing - violated

The space between differential signals clk & clk_n is too big. The max should not exceed 2x trace width.