Information for the THS1206 driver running on the TMS320C6200 and TMS320C6700 DSP family

1	Information	2

1.1	Hardware	2

1.1.1	External Memory Interface (EMIF)	2

1.1.2	Clocking the THS1206	2

1.1.3	The “DATA_AV” pin	3

1.1.4	Base Addresses (THS1206M EVM)	3

1.1.5	Using Multiple THS1206M EVM in one system	3

1.2	Software	4

1.2.1	Used Software	4

1.2.2	Example Project Migration from CCS2.x to CCS3.x	4

2	Hardware Settings	5

2.1	TMS320C6701EVM	5

2.1.1	“Old” THS1206EVM	5

2.1.2	THS1206M EVM using 5-6k I/F board	8

2.2	TMS320C6711DSK / TMS320C6713DSK	11

2.2.1	“Old” THS1206EVM	11

2.2.2	THS1206M EVM using 5-6k I/F board	14

# Information

This driver works for all C62xx and C67xx DSPs and has been tested on the EVM6701, DSK6711 and DSK6713. Block data transfer is done using DMA or EDMA (dependent on which type is supported by the DSP).

Several CSL defines are used for conditional compiling. These defines are set by the CSL dependent on the define CHIP_6xxx in <dc_conf.h>

## Hardware

### External Memory Interface (EMIF)

Emif optimization is done in the function EMIF_init(). The driver selects, dependent on the base address of the converter, the right CE space and sets the CE space control register (CECTLx) appropriate.

The current setup for the CE space where the converter is connected to is:

Setup: 0

Strobe: 4

Hold: 0

This setting works fine for all DSKs and EVM, but further optimization can be made. Nevertheless the EMIF is able to transfer the max. data rate of 6MSPS using FIFO level > 1.

### Clocking the THS1206

The mini EVM lets the user decide if the clock source for the THS1206 comes either from the DSP or from external.

The driver is able to generate a clock using Timer 1 of the DSP. The clock division value is dependent on the DSP frequency (define DSP_FREQ in “dc_conf.h”) and the desired THS1206 system clock (define ADCx_SAMPLE_FREQ in “dc_conf.h”). Since the Timer frequency is generated using the equation:

Timer frequency = DSP_FREQ / 4 / x

it cannot generate every frequency. Therefore it searches for the exact or (if this is not possible) the next lower possible frequency.

### The “DATA_AV” pin

In the THS1206 configuration registers the polarity (high or low) and the mode (pulse or static) of the DATA_AV pin can be selected. The DMA can trigger at the rising edge of the external interrupt pin only (DATA_AV pin of the THS1206 is connected to one of the four available external interrupt pins of the DSP). Therefore it is highly recommended that the mode “high pulse” (bitfields DATA_T, DATA_P both ‘1’ in control register 1) is selected when using block transfer (using DMA) to avoid event delay. Refer to page 29 in document SLAS217H.

With the setup described later different interrupts are used by the DSP platforms (based on the jumper settings below):

EVM6701: 	EXT_INT7 always

DSK6711: 	5-6k I/F board rev. A: EXT_INT4

5-6k I/F board rev. B: EXT_INT5

DSK6711: 	5-6k I/F board rev. A: EXT_INT4

5-6k I/F board rev. B: EXT_INT5

For the EVM6x01 only interrupt #7 is connected to the daughterboard interface.

On the DSKs interrupts can be changed in the data converter plug-in in conjunction with J13 of the C5/6k interface board.

### Base Addresses (THS1206M EVM)

With W4 the base address for the converter can be selected:

TMS320C6701EVM:

W4 position A : 0x01424000

W4 position B : 0x01428000

W4 position C : 0x0142C000

TMS320C6711DSK, TMS320C6713DSK

W4 position A : 0xA0024000

W4 position B : 0xA0028000

W4 position C : 0xA002C000

### Using Multiple THS1206M EVM in one system

With the 5/6k I/F board and the THS1206M EVM it is possible to support up to three boards in the system. This walk-through works with the 5/6k I/F board (rev. B) only. The EVM6701 is not supported since only one interrupt is connected to the daughter board connector.

Board naming:

- Board #1 is the board directly connected to the 5/6k I/F board

- Board #2 is the board on top of Board #1

- Board #3 is the board on top of Board #2

For Board #1 no hardware changes are necessary.

For Board #2 and Board #3 cut the trace going to J2.19 (DATA_AV signal).

For Board #2 connect the just cut trace (the end where the converter is connected to) to J13 pin 8 on the 5/6k I/F board (the right-most pin on the side where the label INT is printed).

For Board #3 connect the just cut trace (the end where the converter is connected to) to W8 pin 2 on the 5/6k I/F board. If W8 has a jumper, remove it.

The jumpers remain the same on all THS1206M EVM board except W4:

Board #1: W4: 1-2

Board #2: W4: 3-4

Board #3: W4: 5-6

When all changes are made the communication is as follows:

TMS320C6711/6713 DSK’s:

Board #1: Base Address: 0xA0024000; Interrupt: EXT_INT5

Board #2: Base Address: 0xA0028000; Interrupt: EXT_INT7

Board #3: Base Address: 0xA002C000; Interrupt: EXT_INT4

Right know the Data converter Support Tool does not support more than one THS1206 in the system. Please adapt you software accordingly.

## Software

### Used Software

All example projects were generated with CCS 2.20.

### Example Project Migration from CCS2.x to CCS3.x

Since this Project Example was created using CCS2.20 the following issue may rise when trying to run it on a CCS3.x

- During Project open

  - Issue: Library DSK6713bsl.lib cannot be located

  - Solution: remove the link from the project and remove the line DSK6713_init() in file main.c

- During Compiling

  - Issue: CDB file version not compatible

  - Solution: Open the CDB file (click yes for conversion) and save the CDB with the same name

# Hardware Settings

  - TMS320C6701EVM

### “Old” THS1206EVM

#### Jumper Settings TMS320C6701EVM

DIP switch: on-on-off-on-off-on-off-on-on-on-on-on

#### Jumper Settings THS1206EVM

J1: 1-2
J2: (connect jumper away from IC6)
J3: 2-5
J4: 2-5
J5: 1-2
J6: 1-2
J7: 1-2
J10: open
J11: closed
J12: 2-3 (next to IC5)
J13: 1-2

#### Connections

Apply an analog sine wave signal to AINP:
f = 10kHz; V(pp) = 4V; V(off) = 0V

#### Startup

Start CCS, load the project and open a graph window
(Menu: View > Graph > Time/Frequency…) with the following settings:

Display Type: Single Time
Start Address: Buffer
Acquisition Buffer Size: 2048
Display Data Size: 2048
DSP Data Type: 16-bit signed integer

Run the program by pressing F5.
Run it a couple if seconds
Stop the program.
CCS updates the graph window where the signal wave is supposed to show up.

#### Screen Shot

The driver of the project you are using was generated with the Data Converter Support Tool (DCP) using the following settings:

The DSP frequency in the DSP tab of the DCP was choosen to be 133MHz.

#### Additional Notes


Per default DSP/BIOS uses Timer0 for the scheduler. If you are using DSP/BIOS and the DSP timer as THS1206 clock source please go in the CDB file to Scheduling > Clock Manager and right-click the item in order to change the timer used by DSP/BIOS toTimer1, because the THS1206EVM is hardwired to Timer0.

### THS1206M EVM using 5-6k I/F board

#### Jumper Settings TMS320C6701

DIP switch: on-on-off-on-off-on-off-on-on-on-on-on

#### Jumper Settings THS1206M EVM

W1: closed
W2: closed
W3: closed
W4: 1-4 (left most pair)
W5: 1-2
W6: 1-2
W7: 1-2 (hardwired)
W8: 1-2 (hardwired)
W9: closed
W10: closed
W11: open

#### Jumper Settings 5-6k I/F board (rev. A)

W1: open
W2: 2-3
W3: 2-3
W4: 2-3
W5: 2-3
W6: see section “Notes” below
J13: TOUTb
J14: a (see section “Notes” below)

Other: don’t care

#### Jumper Settings 5-6k I/F board (rev. B)

W1: open

W2: 2-3

W3: 2-3

W4: 2-3

W5: 2-3

W6: 1-2

W7: 1-2

J13: 7-8 (d)

J14: 3-4 (TOUTb)

Other: don’t care

#### Connections

Apply an analog sine wave signal to AINP: f = 10kHz; V(pp) = 1.3V;
V(off) = 2.5V
Apply +5V to J2 (+5VA(pin 3); GND(pin 2)) of the 5-6k I/F board.

#### Startup

Start CCS, load the project and open a graph window
(Menu: View > Graph > Time/Frequency…) with the following settings:

Display Type: Single Time
Start Address: Buffer
Acquisition Buffer Size: 2048
Display Data Size: 2048
DSP Data Type: 16-bit signed integer

Run the program by pressing F5.
Run it a couple if seconds.
Stop the program.
CCS updates the graph window where the signal wave is supposed to show up.

#### Screenshot

The driver of the project you are using was generated with the Data Converter Support Tool (DCP) using the following settings:

The DSP frequency in the DSP tab of the DCP was chosen to be 133MHz.

#### Additional Notes

When using 5-6k I/F board (rev. A): Due to a hardware problem remove the jumper from W6 and connect pin 2 of W6 to GND.
The external interrupt 7 is the only connected to the expansion port of the C6701EVM. Due to this always choose “Ext Int 7” when using the C6701 EVM. Nevertheless the J13 header of the 5-6k I/F board (responsible for interrupt selection of the DSP) needs to be at position “a”.

## TMS320C6711DSK / TMS320C6713DSK

### “Old” THS1206EVM

#### Jumper Settings TMS320C6711DSK / TMS320C6713DSK

TMS320C6713DSK

SW3: all off

#### Jumper Settings THS1206EVM

J1: 1-2
J2: (connect jumper away from IC6)
J3: 2-5
J4: 2-5
J5: 1-2
J6: 1-2
J7: 1-2
J10: open
J11: closed
J12: 2-3 (next to IC5)
J13: 1-2

#### Connections

Apply an analog sine wave signal to AINP: f = 10kHz; V(pp) = 4V; V(off) = 0V

#### Startup

Start CCS, load the project and open a graph window
(Menu: View > Graph > Time/Frequency…) with the following settings:

Display Type: Single Time
Start Address: Buffer
Acquisition Buffer Size: 2048
Display Data Size: 2048
DSP Data Type: 16-bit signed integer

Run the program by pressing F5.
Run it a couple if seconds
Stop the program.
CCS updates the graph window where the signal wave is supposed to show up.

#### Screenshot

The driver of the project you are using was generated with the Data Converter Support Tool (DCP) using the following settings:

The DSP frequency in the DSP tab of the DCP was chosen to be 150MHz (C6711) or 225MHz (C6713).

#### Additional Notes

Per default DSP/BIOS uses Timer0 for the scheduler. If you are using DSP/BIOS and the DSP timer as THS1206 clock source please go in the CDB file to Scheduling > Clock Manager and right-click the item in order to change the timer used by DSP/BIOS toTimer1, because the THS1206EVM is hardwired to Timer0.

### THS1206M EVM using 5-6k I/F board

#### Jumper Settings TMS320C6711DSK / TMS320C6713DSK

TMS320C6713DSK

SW3: all off

#### Jumper Settings THS1206M EVM

W1: closed
W2: closed
W3: closed
W4: 1-2 (left most pair)
W5: 1-2
W6: 1-2
W7: 1-2 (hardwired)
W8: 1-2 (hardwired)
W9: closed
W10: closed
W11: open

#### Jumper Settings 5-6k I/F board (rev. A)

W1: open
W2: 2-3
W3: 2-3
W4: 2-3
W5: 2-3
W6: see section “Notes” below
J13: TOUTb
J14: a

#### Jumper Settings 5-6k I/F board (rev. B)

W1: open

W2: 2-3

W3: 2-3

W4: 2-3

W5: 2-3

W6: 1-2

W7: 1-2

J13: 1-2 (b) (see the Note in the Screen Shot section!)

J14: 3-4 (TOUTb)

Other: don’t care

#### Connections

Apply an analog sine wave signal to AINP:

f = 10kHz; V(pp) = 1.3V; V(off) = 2.5V
Apply +5V to J2 (+5VA(pin 3); GND(pin 2)) of the 5-6k I/F board.

#### Startup

Start CCS, load the project and open a graph window
(Menu: View > Graph > Time/Frequency…) with the following settings:

Display Type: Single Time
Start Address: Buffer
Acquisition Buffer Size: 2048
Display Data Size: 2048
DSP Data Type: 16-bit signed integer

Run the program by pressing F5.
Run it a couple if seconds
Stop the program.
CCS updates the graph window where the signal wave is supposed to show up.

#### Screenshot

The driver of the project you are using was generated with the Data Converter Support Tool (DCP) using the following settings:

The DSP frequency in the DSP tab of the DCP was chosen to be 150MHz (C6711) or 225MHz (C6713).

IMPORTANT NOTE: If you are using the 5/6k I/F board (rev. B) select Ext Int 5 as DSP interrupt in the GUI.

#### Additional Notes

When using 5-6k I/F board (rev. A): Due to a hardware problem remove the jumper from W6 and connect pin 2 of W6 to GND.