Compare Local variables for psDevice with the report from USBlyzer; the descriptors appear to be garbage for psDevice.

USB Input Device

| Connection Status | Device connected |
| --- | --- |
| Current Configuration | 1 |
| Speed | Full (12 Mbit/s) |
| Device Address | 1 |
| Number Of Open Pipes | 2 |

Device Descriptor PenMount USB

| Offset | Field | Size | Value | Description |
| --- | --- | --- | --- | --- |
| 0 | bLength | 1 | 12h |  |
| 1 | bDescriptorType | 1 | 01h | Device |
| 2 | bcdUSB | 2 | 0110h | USB Spec 1.1 |
| 4 | bDeviceClass | 1 | 00h | Class info in Ifc Descriptors |
| 5 | bDeviceSubClass | 1 | 00h |  |
| 6 | bDeviceProtocol | 1 | 00h |  |
| 7 | bMaxPacketSize0 | 1 | 40h | 64 bytes |
| 8 | idVendor | 2 | 14E1h |  |
| 10 | idProduct | 2 | 6000h |  |
| 12 | bcdDevice | 2 | A4B4h | ?4.?4 |
| 14 | iManufacturer | 1 | 01h | "DIALOGUE INC" |
| 15 | iProduct | 1 | 02h | "PenMount USB" |
| 16 | iSerialNumber | 1 | 00h |  |
| 17 | bNumConfigurations | 1 | 01h |  |

Configuration Descriptor 1 Bus Powered, 500 mA

| Offset | Field | Size | Value | Description |
| --- | --- | --- | --- | --- |
| 0 | bLength | 1 | 09h |  |
| 1 | bDescriptorType | 1 | 02h | Configuration |
| 2 | wTotalLength | 2 | 0029h |  |
| 4 | bNumInterfaces | 1 | 01h |  |
| 5 | bConfigurationValue | 1 | 01h |  |
| 6 | iConfiguration | 1 | 04h | "full speed" |
| 7 | bmAttributes | 1 | A0h | Bus Powered, Remote Wakeup |
|  | 4..0: Reserved |  | ...00000 |  |
|  | 5: Remote Wakeup |  | ..1..... | Yes |
|  | 6: Self Powered |  | .0...... | No, Bus Powered |
|  | 7: Reserved (set to one) (bus-powered for 1.0) |  | 1....... |  |
| 8 | bMaxPower | 1 | FAh | 500 mA |

Interface Descriptor 0/0 HID, 2 Endpoints

| Offset | Field | Size | Value | Description |
| --- | --- | --- | --- | --- |
| 0 | bLength | 1 | 09h |  |
| 1 | bDescriptorType | 1 | 04h | Interface |
| 2 | bInterfaceNumber | 1 | 00h |  |
| 3 | bAlternateSetting | 1 | 00h |  |
| 4 | bNumEndpoints | 1 | 02h |  |
| 5 | bInterfaceClass | 1 | 03h | HID |
| 6 | bInterfaceSubClass | 1 | 00h |  |
| 7 | bInterfaceProtocol | 1 | 00h |  |
| 8 | iInterface | 1 | 03h | "EndPoint1 Interrupt Pipe" |

HID Descriptor

| Offset | Field | Size | Value | Description |
| --- | --- | --- | --- | --- |
| 0 | bLength | 1 | 09h |  |
| 1 | bDescriptorType | 1 | 21h | HID |
| 2 | bcdHID | 2 | 0101h | 1.01 |
| 4 | bCountryCode | 1 | 00h |  |
| 5 | bNumDescriptors | 1 | 01h |  |
| 6 | bDescriptorType | 1 | 22h | Report |
| 7 | wDescriptorLength | 2 | 004Ch | 76 bytes |

Endpoint Descriptor 81 1 In, Interrupt, 1 ms

| Offset | Field | Size | Value | Description |
| --- | --- | --- | --- | --- |
| 0 | bLength | 1 | 07h |  |
| 1 | bDescriptorType | 1 | 05h | Endpoint |
| 2 | bEndpointAddress | 1 | 81h | 1 In |
| 3 | bmAttributes | 1 | 03h | Interrupt |
|  | 1..0: Transfer Type |  | ......11 | Interrupt |
|  | 7..2: Reserved |  | 000000.. |  |
| 4 | wMaxPacketSize | 2 | 0005h | 5 bytes |
| 6 | bInterval | 1 | 01h | 1 ms |

Endpoint Descriptor 02 2 Out, Interrupt, 1 ms

| Offset | Field | Size | Value | Description |
| --- | --- | --- | --- | --- |
| 0 | bLength | 1 | 07h |  |
| 1 | bDescriptorType | 1 | 05h | Endpoint |
| 2 | bEndpointAddress | 1 | 02h | 2 Out |
| 3 | bmAttributes | 1 | 03h | Interrupt |
|  | 1..0: Transfer Type |  | ......11 | Interrupt |
|  | 7..2: Reserved |  | 000000.. |  |
| 4 | wMaxPacketSize | 2 | 0005h | 5 bytes |
| 6 | bInterval | 1 | 01h | 1 ms |

Interface 0 HID Report Descriptor Mouse

| Item Tag (Value) | Raw Data |
| --- | --- |
| Usage Page (Generic Desktop) | 05 01 |
| Usage (Mouse) | 09 02 |
| Collection (Application) | A1 01 |
| Usage (Pointer) | 09 01 |
| Collection (Physical) | A1 00 |
| Usage (Undefined) | 09 00 |
| Logical Minimum (0) | 15 00 |
| Logical Maximum (1) | 25 01 |
| Report Count (6) | 95 06 |
| Report Size (1) | 75 01 |
| Input (Cnst,Ary,Abs) | 81 01 |
| Usage Page (Button) | 05 09 |
| Usage Minimum (Button 1) | 19 01 |
| Usage Maximum (Button 3) | 29 03 |
| Logical Minimum (0) | 15 00 |
| Logical Maximum (1) | 25 01 |
| Report Count (2) | 95 02 |
| Report Size (1) | 75 01 |
| Input (Data,Var,Abs,NWrp,Lin,Pref,NNul,Bit) | 81 02 |
| Usage Page (Generic Desktop) | 05 01 |
| Usage (X) | 09 30 |
| Usage (Y) | 09 31 |
| Logical Minimum (0) | 15 00 |
| Logical Maximum (1023) | 26 FF 03 |
| Physical Minimum (128) | 36 80 00 |
| Physical Maximum (32767) | 46 FF 7F |
| Report Size (16) | 75 10 |
| Report Count (2) | 95 02 |
| Input (Data,Var,Abs,NWrp,Lin,Pref,NNul,Bit) | 81 02 |
| Usage (Undefined) | 09 00 |
| Logical Minimum (0) | 15 00 |
| Logical Maximum (255) | 26 FF 00 |
| Report Size (8) | 75 08 |
| Report Count (5) | 95 05 |
| Feature (Data,Var,Abs,NWrp,Lin,Pref,NNul,NVol,Bit) | B1 02 |
| End Collection | C0 |
| End Collection | C0 |

This report was generated by USBlyzer